| ECC PT1 |  | ECC PT1(21)217 |
| --- | --- | --- |
| ECC PT1 #69 |  |  |
| 13-17 September 2021 |  |  |
| Date issued: | 8 September 2021 |  |
| Source: | Denmark, Finland, Lithuania, Slovenia, Sweden, UK |  |
| Subject: | Draft CEPT Brief on Article 21.5 |  |
| N Group membership required to read? (Y/N) |  |  |
| Summary: |  |  |
| At the last meeting of ECC PT1 progress has been made mainly on the preliminary CEPT position for Issue B. The last CPG23 meeting made also some further progress (including issue A and C). However neither of those meetings progressed on the Background section of the CEPT Brief which currently contains a number of unresolved comments. This contribution proposes to restructure the Background section with appropriate headings (General, Issue A, Issue B and Issue C). At the last meeting of ITU-R WP5D (June 2021), information was provided on how the limits in Article 21.5 were arrived at. WP5D is also studying two approaches, i.e. “TRP with a reference bandwidth” and “Conducted power delivered by a single transmitter”. For the TRP approach, WP5D is studying various reference bandwidths (1 MHz, up to a few tens of MHz and 200 MHz). It is important that ECC PT1 carefully considers the work being carried out in WP5D including the appropriate reference bandwidth to be applied. For Issue B, this contribution proposes that the reference bandwidth should be reflective of the studies carried out when limit in Article 21.5 was introduced in 1963 and 1970. A typical channel bandwidth of 200 MHz used in the studies leading to WRC-19 has no justification to the protection to satellite and could unnecessarily constrain IMT deployment. For Issue A, this contribution proposes that the limit/reference bandwidth agreed under Issue B needs to be analysed to see if it is suitable, or needs to be adapted for, other bands agreed under Issue C. It should not impact the protection of satellite reception or constrain the deployment of IMT stations or other Fixed and Mobile services. An updated CEPT Brief is provided in the Annex. This contribution does not comment on Issue C which is under the remit of CPG PTB. |  |  |
| Proposals: |  |  |
| - invites PT1 to consider the attached changes to the draft CEPT Brief. New changes are highlighted in cyan. |  |  |
| Background: |  |  |
| At the last meeting of ITU-R WP5D in June 2021, CCIR Recommendation 406, CCIR Report 209 and CCIR Report 450 were reviewed in conjunction with the BR informal document provided to the WP5D October 2020 meeting, and a brief summary of these documents was made available. It was found from the BR informal document that, an input from the Special Joint Meeting of CCIR to World Administrative Radio Conference for Space Telecommunications in 1971 pointed out that “all power limitations should be stated without reference to a bandwidth unit” and “the concept of defining e.i.r.p. and transmitter power on the basis of power per unit bandwidth merits further consideration by the CCIR”. The original studies to determine the limits of Article 21.5 were carried out at 4 GHz and 11 GHz (CCIR Report 450). WP5D is studying two approaches, i.e. “TRP with a reference bandwidth” and “Conducted power delivered by a single transmitter”. For the TRP approach, WP5D has received inputs to study following reference bandwidths: - 200 MHz - 1 MHz - up to a few tens of MHz, such as 28 or 56 MHz |  |  |

Annex: updated draft CEPT Brief on Article 21.5