| ECC PT1 |  |  | ECC PT1(22)028 |  |
| --- | --- | --- | --- | --- |
| ECC PT1 #70 |  |  |  |  |
| Web meeting, 11-12 and 17-21 January 2022 |  |  |  |  |
| Date issued: | 5 January 2022 |  |  |  |
| Source: | Lithuania |  |  |  |
| Subject: | Proposed amendments for ECC Recommendation (15)01 |  |  |  |
| Group membership required to read? (Y/N) |  | N |  |  |
| Summary: |  |  |  |  |
| During correspondence group (CG) further activity was identified regarding clean-up of the document and finding suitable terminology for the titles of sections on operation modes in Annex 1 of the working document (WD). This input contribution proposes modification to the CG output version of WD. In general they are more of editorial and formatting nature to make that Annex 1 more readable. This includes new headings in Annex 1 for each of subsection, addition of relevant tables summarising every operation mode field strength values. In addition it is proposed to add two new tables at the beginning of section A1.3 which would provide information on the operation modes for 3400-3800 MHz frequency band. It should be noted that many rejections of new proposals shown as deleted in the output version of CG were accepted (i.e. in A1.3 and at the end of A5) since for PC only modifications made for the current version of recommendation are relevant. Although some parts were already highlighted in green during CG it was felt that some elements needed alignment with the new terminology proposed. In order to ease the process other editorial revisions were also done throughout the WD (i.e. correction of formatting in Annex 5 which got distorted during the process; adjustment of formatting for recommends 2-4). Annex to this input contribution highlights new formatting of the sections of Annex 1 of ECC Recommendation (15)01. All the modifications with track changes can be found in the attachment to this input contribution. |  |  |  |  |
| Proposal: |  |  |  |  |
| invites Group to - consider and agree proposed restructure of Annex 1 together with other amendments included in the attachment to this input contribution |  |  |  |  |
| Background: |  |  |  |  |
| ECC PT1 work item PT1_43 on update of ECC Recommendation (15)01 on 3.4-3.8 GHz part taking into account ECC Recommendation (20)03 and draft ECC Report 331 on “Efficient usage of the spectrum at the border of CEPT countries between MFCN TDD networks in the frequency band 3400-3800 MHz”. ECC PT1 meeting #69 agreed to have correspondence group (CG) to address updates to ECC Recommendation (15)01. |  |  |  |  |

Annex: Field strength levels for the cross-border COORDINATION between MFCN systems

- ’Conditions are simplified for presentation purposes, for full description see attachment to the input document

  - Field strength levels in the 700 MHz frequency band

    - FDD case

synchronisation signal centre frequencies not aligned for all PCIs or synchronisation signal centre frequencies aligned for pref. PCIs

59 dBµV/m/5 MHz at a height of 3 m above ground at the borderline between countries and

41 dBµV/m/5 MHz at a height of 3 m above ground at a distance of 6 km inside the neighbouring country

synchronisation signal centre frequencies aligned for non-pref. PCIs

41 dBµV/m/5 MHz at a height of 3 m above ground at the borderline between countries

    - SDL case

irrespective of synchronisation signal centre frequencies alignment or PCIs

59 dBµV/m/5 MHz at a height of 3 m above ground at the borderline between countries and

41 dBµV/m/5 MHz at a height of 3 m above ground at a distance of 6 km inside the neighbouring country

The following Table 1 gives an overview of the field strength values.

Table 1: Field strength levels at a height of 3 m above ground between wideband systems

| Case | Wideband system vs. Wideband system |  |  |
| --- | --- | --- | --- |
|  | Synchronisation signal centre frequencies aligned |  | Synchronisation signal centre frequencies not aligned |
|  | Preferential PCIs | Non-preferential PCIs | All PCIs |
| FDD case | 59 dBμV/m/5 MHz @ 0 km and 41 dBμV/m/5 MHz @ 6 km | 41 dBμV/m/5 MHz @ 0 km | 59 dBμV/m/5 MHz @ 0 km and 41 dBμV/m/5 MHz @ 6 km |
| SDL case | 59 dBμV/m/5 MHz @ 0 km and 41 dBμV/m/5 MHz @ 6 km |  |  |
| @ stands for “at a distance from the borderline into the neighbouring country” |  |  |  |

  - Field strength levels in the 1427-1517 MHz frequency band

    - SDL case

irrespective of synchronisation signal centre frequencies alignment or PCIs

65 dBµV/m/5 MHz at a height of 3 m above ground at the borderline between countries and

47 dBµV/m/5 MHz at a height of 3 m above ground at a distance of 6 km inside the neighbouring country

The following Table 2 gives an overview of the field strength values.

Table 2: Field strength levels at a height of 3 m above ground between wideband systems

| Case | Wideband system vs. Wideband system |
| --- | --- |
| SDL case | 65 dBμV/m/5 MHz @ 0 km and 47 dBμV/m/5 MHz @ 6 km |
| @ stands for “at a distance from the borderline into the neighbouring country” |  |

  - Field strength levels IN THE 3400-3800 MHz frequency band

Table 3: Operation modes

| Operation mode | Operation mode condition | Mode in ECC Report 331 |
| --- | --- | --- |
| Synchronised | - common phase clock reference - compatible frame structures | Mode #1 |
| Unsynchronised with partial duplex misalignment and ECC Recommendation (20)03 recommended frame structures with DSB | - common phase clock reference - frame structure "Frame A" or "Frame B - blanked downlink symbols (of each network that correspond to simultaneous uplink transmissions or simultaneous gap symbols for the other network) | Mode #2 |
| Unsynchronised with partial duplex misalignment and ECC Recommendation (20)03 recommended frame structures without DSB | - common phase clock reference - frame structure “Frame A” or “Frame B” - no blanked downlink symbols | Mode #3 |
| Fully unsynchronised operation with non-preferential frequency blocks | - no common phase clock reference - compatible or non-compatible frame structure | Mode #4 |
| Fully unsynchronised operation with preferential frequency blocks | - no common phase clock reference - compatible or non-compatible frame structure | Mode #5 |
| DSB implementation zone | DSB feature activated | See section 3.1.2 |
| Note: the situation with common phase clock reference and non-compatible frame structures was not addressed in ECC Report 331 |  |  |

Table 4: Cases covered for TDD MFCN operation in the 3400-3800 MHz frequency band

| Case | Country A | Country B | Section |
| --- | --- | --- | --- |
| Common phase clock and compatible frame | Frame X | Frame X | A1.3.1 |
| Common phase clock and frames A and B | Frame A w/ DSB | Frame B w/ DSB | A1.3.2 |
|  | Frame A w/ DSB | Frame B w/o DSB |  |
|  | Frame A w/o DSB | Frame B w/o DSB | A1.3.3 |
| Common phase clock and non-compatible frame and non-pref. block | Frame X | Frame Y | A1.3.4 |
| Common phase clock and non-compatible frame and pref. block | Frame X | Frame Y | A1.3.5 |
| No common phase clock and non-pref. block | Frame X or Y | Frame Y | A1.3.4 |
| No common phase clock and pref. block | Frame X or Y | Frame Y | A1.3.5 |
| w/ stands for “with” and w/o stands for “without” |  |  |  |

    - Synchronised operation

- ’all BS adopt a common phase clock reference and a compatible frame structures

- ’mode #1 values

synchronisation signal centre frequencies not aligned for all PCIs or synchronisation signal centre frequencies aligned for pref. PCIs

[xx] dBµV/m/5 MHz at a height of 3 m above ground at the borderline between countries and

[yy] dBµV/m/5 MHz at a height of 3 m above ground at a distance of 6 km inside the neighbouring country

synchronisation signal centre frequencies aligned for non-pref. PCIs

[yy] dBµV/m/5 MHz at a height of 3 m above ground at the borderline between countries

The following Table 5 gives an overview of the field strength values.

Table 5: Field strength values for synchronised operation

| Synchronised operation |  |  |  |
| --- | --- | --- | --- |
| Synchronisation signal centre frequency aligned |  | Synchronisation signal centre frequency not aligned |  |
| Preferential PCIs | Non-preferential PCIs |  | All PCIs |
| [xx] dBμV/m/5 MHz @ 0 km and [yy] dBμV/m/5 MHz @ 6 km | [yy] dBμV/m/5 MHz @ 0 km |  | [xx] dBμV/m/5 MHz @ 0 km and [yy] dBμV/m/5 MHz @ 6 km |
| @ stands for “at a distance from the borderline inside the neighbouring country” For base station using AAS the value of [xx] dBμV/m/5 MHz correspond to SSB field strength level of a) [zz] dBµV/m//30 kHz for single-beam antenna pattern, b) [ww] dBµV/m/30 kHz for multi-beam antenna pattern For base station using AAS the value of [yy] dBμV/m/5 MHz correspond to SSB field strength level of a) [zz] dBµV/m//30 kHz for single-beam antenna pattern, b) [ww] dBµV/m/30 kHz for multi-beam antenna pattern |  |  |  |

    - Operation with common phase clock reference and frames recommended by ECC Recommendation (20)03 and with DSB

- ’BS adopt a common phase clock reference and frames A or B, and DSB

- ’Description of DSB implementation zone (DSB IZ)

- ’mode #2 values outside DSB IZ; mode #1 values inside DSB IZ

Inside DSB IZ:

Apply A1.3.1

Outside DSB IZ:

irrespective of synchronisation signal centre frequencies alignment and for all PCIs

[yy] dBµV/m/5 MHz at a height of 3 m above ground at the borderline between countries

The following Table 6 gives an overview of the field strength values.

Table 6: Field strength values for operation with common phase clock reference and frames recommended by ECC Recommendation (20)03 and with DSB

| Operation with common phase clock reference and frames recommended by ECC Recommendation (20)03 and with DSB |  |
| --- | --- |
| Inside DSB implementation zone | apply A1.3.1 |
| Outside DSB implementation zone | [yy] dBμV/m/5 MHz @ 0 km for all PCIs |
| @ stands for “at a distance from the borderline into the neighbouring country” [add relevant SSB values accordingly, similar as in Table 5] |  |

    - Operation with common phase clock reference and frames recommended by ECC Recommendation (20)03 and without DSB

- ’all BS adopt a common phase clock reference and frames A or B, no DSB

- ’mode #3 values

all PCIs

[yy] dBµV/m/5 MHz at a height of 3 m above ground at the borderline between countries

The following Table 7 gives an overview of the field strength values.

Table 7: Field strength values for operation with common phase clock reference and frames recommended by ECC Recommendation (20)03 and without DSB

Operation with common phase clock reference and frames recommended by ECC Recommendation (20)03 and without DSB

All PCIs

[yy] dBμV/m/5 MHz @ 0 km

@ stands for “at a distance from the borderline into the neighbouring country” [add relevant SSB values accordingly, similar as in Table 5]

    - Fully-unsynchronised operation with non-preferential frequency blocks

- ’all BSs do not adopt a common phase clock reference and/or compatible/non-compatible frame structure

- ’mode #4 values

all PCIs

[yy] dBµV/m/5 MHz at a height of 3 m above ground at the borderline between countries

The following Table 8 gives an overview of the field strength values.

Table 8: Field strength values for fully-unsynchronised operation with non-preferential frequency blocks

Fully-unsynchronised operation with non-preferential blocks

All PCIs

[yy] dBμV/m/5 MHz @ 0 km

@ stands for “at a distance from the borderline into the neighbouring country” [add relevant SSB values accordingly, similar as in Table 5]

    - Fully-unsynchronised operation with preferential frequency blocks

- ’all BSs do not adopt a common phase clock reference and/or compatible/non-compatible frame structure

- ’mode #5 values

all PCIs

[xx] dBµV/m/5 MHz at a height of 3 m above ground at the borderline between countries and

[yy] dBµV/m/5 MHz at a height of 3 m above ground at a distance of 6 km inside the neighbouring country

The following Table 9 gives an overview of the field strength values.

Table 9: Field strength values for fully-unsynchronised operation with preferential frequency blocks

Fully-unsynchronised operation with preferential blocks

All PCIs

[xx] dBμV/m/5 MHz @ 0 km and [yy] dBμV/m/5 MHz @ 6 km

@ stands for “at a distance from the borderline into the neighbouring country” [add relevant SSB values accordingly, similar as in Table 5]