| Radiocommunication Study Groups |  |
| --- | --- |
| Received: 22 November 2021 Source: Document 5B/355 (Annex 16) | Document 5B/452-E |
|  | 22 November 2021 |
|  | English only |
| Australia, Canada, Croatia (Republic of), Estonia, France, Germany (Federal Republic of), Latvia (Republic of), Lithuania (Republic of), Luxembourg, Netherlands (Kingdom of the), Poland (Republic of), Korea (Republic of), Slovenia (Republic of), Turkey, United Kingdom of Great Britain and Northern Ireland |  |
| PRELIMINARY DRAFT REVISION TO RECOMMENDATION ITU-R M.2116-0 |  |
| Technical characteristics and protection criteria for the systems operating in the aeronautical mobile service and maritime mobile service within the 4 400-4 990 MHz frequency range |  |

(2018-20xx)

[Scope

This Recommendation provides information on the technical characteristics and protection criteria for systems operating in the aeronautical mobile service (AMS) and maritime mobile service (MMS) planned to or currently operating within the frequency range 4 400-4 990 MHz for use in sharing and compatibility studies as needed and does not contain any aeronautical mobile telemetry system.

Keywords

Aeronautical mobile service, maritime mobile service, technical characteristics, protection criteria

Abbreviations/Glossary

ADL	Aeronautical mobile service data link

AMS	Aeronautical mobile service

MDL	Maritime mobile service data link

MMS	Maritime mobile service

Related ITU-R Recommendations and Reports

Recommendation ITU-R M.1851 – Mathematical models for radiodetermination radar systems antenna patterns for use in interference analyses

The ITU Radiocommunication Assembly,

considering

a)	that systems and networks operating in the aeronautical mobile service (AMS) and in the maritime mobile service (MMS) are used for broadband, data-links including aircraft to aircraft links or ship to aircraft links, to support various applications including remote sensing, e.g. earth sciences, land management, energy distribution,  [including in international airspace in accordance with the radio regulations];

c)	that systems and networks operating in AMS and MMS are also used for narrow-band, airborne data-links;

d)	that the physics of the propagation of electromagnetic energy, the availability of hardware components, etc., within the 4 400-4 990 MHz frequency range facilitates the use of current or planned operating systems and networks for applications referenced in considering a), b) and c),

recognizing

a)	that the frequency range 4 400-4 990 MHz is allocated on a primary basis in all three ITU regions to the mobile service;

b)	that other radio services are allocated on either a primary or secondary basis in all or parts of the frequency range 4 400-4 990 MHz all three ITU Regions;

c)	that the RR No. 5.442 provides some restrictions for the use of AMS in parts of the frequency band;

d)	that technical characteristics and protection criteria for [aeronautical mobile telemetry] [and maritime mobile telemetry] systems are not contained in this Recommendation,

recommends

1	that the technical characteristics and protection criteria for systems operating in the AMS given in the Annex 1 should be used in performing sharing and compatibility analyses.

2	that the technical characteristics and protection criteria for systems operating in the MMS given in Annex 2 should be used performing sharing and compatibility analyses.

3	that the following Note is considered as part of this Recommendation.

NOTE – The characteristics and protection criteria should not have any adverse effect to Appendix 30B of the Radio Regulations]

Annex 1

Technical characteristics and protection criteria for systems operating in the aeronautical mobile service

[Editor’s note: these previous topics expressed in both views are not of a technical nature and should be addressed in the WD Report on CONDITIONS 1.1]

[Editor’s note: these technical characteristics are considered applicable independently whether the systems are operated in international water/airspace or in national areas]

# 1	Introduction

Systems and networks operating in the AMS are used for broadband, airborne data-links including aircraft to aircraft, to support various applications as remote sensing, e.g. earth sciences, land management, energy distribution.

These aeronautical mobile systems (uplink, downlink and air-air) operate on a 24/7 basis to support security, law enforcement, and humanitarian assistance efforts throughout the 4 800-4 990 MHz frequency range in their own territories. Some operations can also take place in international airspace and waters (e.g. to fight against piracy, to support humanitarian operations, to escort ships, for deep sea rescue, for search and rescue/emergency operations at sea, etc).

It can be single link involving AMS (and/or MMS) stations or a mesh networks involving several AMS stations (and/or MMS) stations.

# 2	Operational deployment

[Editor’s note: see proposal below]

Aeronautical mobile data links are operated between aeronautical stations and aircraft stations, or between aircraft stations or ship stations equipped with AMS data links (ADL) and can be deployed anywhere except within countries whose administration has not authorized their use.

Some of these operations in the international aerospace can be planned in advance, whereas some other operations may take place in unpredictable time and location.

ADL includes transmission from and to, either aircraft stations or a ground terminal considered as an aeronautical station. These transmissions could use bidirectional air-to-ground links, or relay through another airborne platform using an air-to-air data link. Links can be either simplex or duplex. The link lengths vary greatly in these applications. The link lengths may approach the radio line-of-sight distance in order to ensure the largest coverage as possible in particular in international zones. The operational altitude of airborne platforms equipped with these ADLs can vary from ground/sea level to up to 20 000 m.

The ground terminals may be at a permanent location or they may be transportable. Transportable ground terminals can be moved to meet operational needs and the duration of use while it remains at a particular location is dependent upon operational requirements. In certain instances, an aeronautical station may be located, for example, on board ship or on a platform at sea.

[Editor’s note: to fixe editorially after the meeting]

A single ground terminal may simultaneously support several aircraft stations at the same time via different links.

The application of system 6 is an automated unmanned aerial vehicle (UAV) based wide area ocean surface exploration system used to conduct multiple activities including maritime search and rescue, disaster relief support activities and support to air crash investigations conducted in territorial and international waters. The system consists of multiple UAVs conducting video surveillance of a wide ocean surface area. In order to achieve the required coverage that satisfies large video surveillance footprints, the UAVs form a mesh network to deliver high resolution video to either a ship or land based command and monitoring centers. The received video data is used to identify objects of interest, such as, aircraft debris and distressed personnel. The frequency selection for individual UAVs depends on the number of UAVs participating in a task and their bandwidth requirements. The mesh network can be configured in multiple ways depending on the task requirements, either as a single network or multiple sub-networks assigned with dedicated frequency channels and bandwidths. Following figure depicts the above mentioned application. Table 1 contains the characteristics of the radio systems used for payload communications. It should be noted that Table 1 only depicts radio systems used for payload communications as part of this application and those used for non-payload communications are not indicated in this table. In Table 1 for System 6, Airborne 1 and Airborne 2 represent two UAVs with similar radio system characteristics and are used to identify two ends of a single hop communication link within the mesh network.

Figure 1

Operation of UAV based wide area ocean surface exploration system

[Editor’s  note: in the methodology considered within WP 5D, the assessed interference is to a AMS or MMS station independently it is a part of mesh networks or a single station.]

The application of System 7 in Table 1 is earth surface exploration system used to conduct or support activities including maritime search and rescue, disaster relief and rescue in national territories and international [airspace and] waters. Once the visual monitoring results are taken by any aircraft, the captured video is delivered from one aircraft to the other by using 5 MHz aeronautical datalink (ADL) and any audio communication between aircrafts is delivered by using 8 kHz ADL as depicted in Figure 2. The details of technical characteristics are given in Table 1. The center frequency for two ADLs will be selected differently in the tuning range. In Figure 2 two aircrafts are operating in one set. There could be multiple sets.

Figure 2

Example of configuration of two aeronautical datalinks by System 7

[Editor’s note: the introduction part was adjusted to reflect this application.]

The System 8 is planned for use both on national territory and in international airspace and international waters.

The main application of this system:

–	exchange of various information, including the transfer of high-speed data, with aircraft and ships performing various commercial and science missions;

–	organization of monitoring of linear and area hazardous production facilities and areas.

Direct communication between aircrafts and ships is also possible.

With regard to international waters and international airspace, the use of this system is intended only in local areas for conducting pre-planned research missions, for example, scientific studies of the sea surface or the atmosphere.

The construction of this system is planned on the basis of modern commercially available state-of-art telecommunication equipment.

# 3	Technical characteristics of aeronautical mobile systems

Typical technical characteristics for representative airborne data links for the frequency range 4 400-4 990 MHz are provided in Table 1.

## 3.1	Transmitter and receiver characteristics

The aeronautical mobile systems operating or planned to operate within the frequency range 4 400-4 990 MHz typically use digital modulations. A given transmitter may be capable of radiating more than one waveform.

## 3.2	Antenna characteristics

A variety of different types of antennas are used by systems in the frequency range 4 400-4 990 MHz. Antennas in this range are generally of a variety of sizes and vary between the airborne component of the link and the ground based component of the link. The airborne antenna gains are typically between +3 dBi and 19 dBi. The ground based antenna gain is typically between 3 dBi and 31 dBi. Horizontal, and vertical polarizations could be used.

Antenna characteristics available in Table 1 should be used for studies unless measured data is available.

# 4	Protection criteria

An increase in receiver effective noise of 1 dB would result in significant degradation in communication range.

Such an increase in effective receiver noise level corresponds to an (I + N)/N ratio of 1.26, or an I/N ratio of about −6 dB. This represents the required protection criterion for the AMS systems referenced herein from interference due to another radiocommunication service. If multiple potential interference sources are present, protection of the AMS and MMS systems requires that this criterion is not exceeded due to the aggregate interference from the multiple sources.

TABLE 1

Typical technical characteristics of representative systems operating in the aeronautical mobile service 
in the frequency range 4 400-4 990 MHz

| Parameter | Units | System 1 Airborne | System 1 Ground |  |  | System 2 Airborne | System 2 Ground |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |  |  |  |  |
| Tuning range | MHz | 4 400-4 990(1) | 4 400-4 990(1) |  |  | 4 400-4 990(1) | 4 400-4 990(1) |  |  |
| Power output | dBm | 45 | 45 |  |  | 35-39 | 30-39 |  |  |
| Bandwidth (3 dB) | MHz | 1 | 1 |  |  | 6 / 10 / 20 | 6 / 10 / 20 |  |  |
| Receiver |  |  |  |  |  |  |  |  |  |
| Tuning range | MHz | 4 400-4 990(1) | 4 400-4 990(1) |  |  | 4 400-4 990(1) | 4 400-4 990(1) |  |  |
| Selectivity (3 dB) | MHz | 1 | 1 |  |  | 6 / 10 / 20 | 6 / 10 / 20 |  |  |
| Noise figure | dB | 3.5 | 3 |  |  | 3.5 | 3 |  |  |
| Thermal noise level | dBm | −110.5 | −111 |  |  | −102.5 to −97.5 | −103 to −98 |  |  |
| Antenna |  |  |  |  |  |  |  |  |  |
| Antenna type |  | Omnidirectional | Omni-directional | Directional |  | Omnidirectional | Omni-directional | Directional |  |
| Antenna gain | dBi | 3 | 3 | 19 | 31 | 3 | 6 | 19 | 31 |
| 1st sidelobe | dBi | N/A(2) | N/A(2) | 6 | 11 | N/A(2) | N/A(2) | 6 | 11 |
| Polarization |  | Vertical | Vertical | Vertical |  | Vertical | Vertical | Vertical |  |
| Antenna pattern |  | N/A(2) | N/A(2) | Uniform distribution(3) |  | N/A(2) | N/A(2) | Uniform distribution(3) |  |
| Horizontal beamwidth | Degrees | 360 | 360 | 16 | 3.3 | 360 | 360 | 16 | 3.3 |
| Vertical beamwidth | Degrees | 90 | 90 | 16 | 3.3 | 90 | 90 | 16 | 3.3 |

TABLE 1 (Continued)

| Parameter | Units | System 3 Airborne |  | System 3 Ground and shipborne |  | System 4 Airborne |  | System 4 Ground |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |  |  |  |  |
| Tuning range | MHz | 4 400-4 940(1) |  | 4 400-4 940(1) |  | 4 400-4 940(1) |  | 4 400-4 940(1) |  |
| Power output | dBm | 42-50 |  | 42 |  | 43 |  | 37 |  |
| Bandwidth (3 dB) | MHz | 0.158 / 0.97 / 1.23 / 4.0 |  | 0.158 / 0.97 / 1.23 / 4.0 |  | 0.158 / 2.4 / 4.8 / 9.6 |  | 0.158 / 2.4 / 4.8 / 9.6 |  |
| Receiver |  |  |  |  |  |  |  |  |  |
| Tuning range | MHz | 4 400-4 940(1) |  | 4 400-4 940(1) |  | 4 400-4 940(1) |  | 4 400-4 940(1) |  |
| Selectivity (3 dB) | MHz | 0.2 / 1 / 1.5 / 4.5 |  | 0.2 / 1 / 1.5 / 4.5 |  | 0.2 / 2.6 / 5.0 / 10 |  | 0.2 / 2.6 / 5.0 / 10 |  |
| Noise figure | dB | 2.5 |  | 2.5 (ground) / 6 (shipborne) |  | 2.5 |  | 3 |  |
| Thermal noise level | dBm | 118.5 to 105.0 |  | 118.5 to 105.0 |  | 118.5 to 101.5 |  | 118 to 101 |  |
| Antenna |  |  |  |  |  |  |  |  |  |
| Antenna type |  | Omni-directional | Directional | Omni-directional | Directional | Omni-directional | Directional | Omni-directional | Directional |
| Antenna gain | dBi | 3.5 | 16 | 3 | 30 | 4.5 | 16 | 4 | 30 |
| 1st sidelobe | dBi | N/A(2) | 9 | N/A(2) | 17 | N/A(2) | 9 | N/A(2) | 17 |
| Polarization |  | Vertical | Vertical | Vertical | Vertical | Vertical | Vertical | Vertical | Vertical |
| Antenna pattern |  | N/A(2) | Uniform distribution(3) | N/A(2) | Uniform distribution(3) | N/A(2) | Uniform distribution(3) | N/A(2) | Uniform distribution(3) |
| Horizontal beamwidth | degrees | 360 | 33 | 360 | 4.4 | 360 | 33 | 360 | 4.4 |
| Vertical beamwidth | degrees | 35 | 33 | 40 | 4.4 | 35 | 33 | 60 | 4.4 |

TABLE  1 (Continued)

| Parameter | Units | System 5 Airborne |  | System 5 Ground [and shipborne] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |  |
| Tuning range | MHz | 4 400-4 990(1) |  | 4 400-4 990(1) |  |  |
| Power output | dBm | 45 |  | 45 |  |  |
| Bandwidth (3 dB) | MHz | 0.4 / 3 / 8.5 |  | 0.4 / 3 / 8.5 |  |  |
| Receiver |  |  |  |  |  |  |
| Tuning range | MHz | 4 400-4 990(1) |  | 4 400-4 990(1) |  |  |
| Selectivity (3 dB) | MHz | 0.4 / 3 / 17 |  | 0.4 / 3 / 17 |  |  |
| Noise figure | dB | 3.5 |  | 3.5 (ground) / [6 (shipborne)] |  |  |
| Thermal noise level | dBm | 118.5 to 105.0 |  | 118.5 to 105.0 |  |  |
| Antenna |  |  |  |  |  |  |
| Antenna type |  | Omni-directional | Directional | Omni-directional | Directional |  |
| Antenna gain | dBi | 3 | 19 | 3 | 19 | 31 |
| 1st sidelobe | dBi | N/A(2) | 6 | N/A(2) | 6 | 11 |
| Polarization |  | Vertical | Vertical | Vertical | Vertical |  |
| Antenna pattern |  | N/A(2) | see equation in (4) | N/A(2) | [see equation in (4) (5) /Uniform distribution(3) ] |  |
| Horizontal beamwidth | degrees | 360 | 16 | 360 | 16 | 3.3 |
| Vertical beamwidth | degrees | 90 | 16 | 360 | 16 | 3.3 |
| Notes: (1) RR No. 5.442 applies. (2) N/A – Not applicable. (3) Refer to Recommendation ITU-R M.1851. (4) For antenna gain 19 dBi:Gψ=20.log10sinc3.19πsinψ+19.0∀ψ∈−68.43°,68.43° and Gψ=−20 otherwise. Here, sincx=sinxx∀x≠0 (x in radians) and sinc0=1. (5) For antenna gain 31 dBi: Gψ= 20.log10𝑠𝑖𝑛𝑐15.5𝜋sin𝜓+31.0 ∀ψ∈−64.25°,64.25° and Gψ=−20 otherwise. Here, sincx=sinxx∀x≠0 (x in radians) and sinc0=1. [Editor’s note: the need of this equation should be confirmed. One possible solution is to keep using footnote (3) in case of uniform distribution] In the Table “-“ means range of values, and “/” means discrete values. [Editor’s note: the noise figure in some parts of Table 1 needs to be further clarified] |  |  |  |  |  |  |

TABLE  1 (Continued)

| Parameter | Units | System 6 Airborne 1 | System 6 Airborne 2 | System 6 Ship borne |  | System 6 Ground |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |  |  |
| Tuning range | MHz | 4 800-4 990 | 4 800-4 990 | 4 800-4 990 |  | 4 800-4 990 |  |
| Power output | dBm | 27-33 | 27-33 | 35 |  | 35 |  |
| Bandwidth (3 dB) | MHz | 5/10/20/40 (software configurable) | 5/10/20/40 (software configurable) | 5/10/20/40 (software configurable) |  | 5/10/20/40 (software configurable) |  |
| Receiver |  |  |  |  |  |  |  |
| Tuning range | MHz | 4 800-4 990 | 4800-4 990 | 4 800-4 990 |  | 4 800-4 990 |  |
| Selectivity (3 dB) | MHz | 5/10/20/40 | 5/10/20/40 | 5/10/20/40 |  | 5/10/20/40 |  |
| Noise figure | dB | 6 | 6 | 6 |  | 4 |  |
| Thermal noise level | dBm | −101 to -92 | −101 to -92 | −103 to −94 |  | −103 to −94 |  |
| Antenna |  |  |  |  |  |  |  |
| Antenna type |  | Omnidirectional | Omnidirectional | Omni-directional | Directional | Omni-directional | Directional |
| Antenna gain | dBi | 4.7 | 4.7 | 6 | 11.8 | 6 | 11.8 |
| 1st sidelobe | dBi | N/A | N/A | N/A | Note 2 | N/A | Note 2 |
| Polarization |  | Vertical | Vertical | Vertical | Vertical | Vertical | Vertical |
| Antenna pattern |  | N/A | N/A | Note 1 | Note 2 | Note 1 | Note 2 |
| Horizontal beamwidth | Degrees | 360 | 360 | 360 | 30 | 360 | 30 |
| Vertical beamwidth | Degrees | 90 | 90 | 28 | 18 | 28 | 18 |

TABLE  1 (Continued)

| Parameter | Units | System 7 Airborne 1 | System 7 Airborne 2 |
| --- | --- | --- | --- |
| Transmitter |  |  |  |
| Tuning range | MHz | 4 400-4 990 | 4 400-4 990 |
| Power output | dBm | 30-43 | 30-43 |
| Bandwidth (3 dB) | MHz | 5 / 0.008 | 5 / 0.008 |
| Receiver |  |  |  |
| Tuning range | MHz | 4 400-4 990 | 4 400-4 990 |
| Selectivity (3 dB) | MHz | 5 / 0.008 | 5 / 0.008 |
| Noise figure | dB | 6 | 6 |
| Thermal noise level | dBm | -103 / −131 | -103/ −131 |
| Antenna |  |  |  |
| Antenna type |  | Directional | Directional |
| Antenna gain | dBi | 14 | 14 |
| 1st sidelobe | dBi | -1 | -1 |
| Polarization |  | Vertical | Vertical |
| Antenna pattern |  | Uniform distribution (Refer to Rec. ITU-R M.1851) | Uniform distribution (Refer to Rec. ITU-R M.1851) |
| Horizontal beamwidth | Degrees | 24 | 28 |
| Vertical beamwidth | Degrees | 24 | 28 |

TABLE 1 (End)

| Parameter | Units | System 8 Airborne | System 8 Ground | System 8 Shipborne |
| --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |
| Tuning range | MHz | 4 800-4 990 | 4 800-4 990 | 4 800-4 990 |
| Power output | dBm | 26 | 46 | 46 |
| Bandwidth (3 dB) | MHz | 40/50/60/80/100 (software configurable) | 40/50/60/80/100 (software configurable) | 40/50/60/80/100 (software configurable) |
| Receiver |  |  |  |  |
| Tuning range | MHz | 4 800-4 990 | 4 800-4 990 | 4 800-4 990 |
| Selectivity (3 dB) | MHz | 40/50/60/80/100 | 40/50/60/80/100 | 40/50/60/80/100 |
| Noise figure | dB | 9 | 5 | 5 |
| Thermal noise level | dBm | −89 … -85 | −93 … -89 | −93 … -89 |
| Antenna |  |  |  |  |
| Antenna type |  | Omnidirectional | Directional (steerable, MIMO) | Directional (steerable, MIMO) |
| Antenna gain | dBi | 0 | 15 | 15 |
| 1st sidelobe | dBi | N/A | N/A | N/A |
| Polarization |  | Vertical | Vertical | Vertical |
| Antenna pattern |  | N/A | Rec ITU-R F.1336 | Rec ITU-R F.1336 |
| Horizontal beamwidth | Degrees | 360 | 65 | 65 |
| Vertical beamwidth | Degrees | 90 | 90 | 90 |

Annex 2

Technical characteristics and protection criteria for systems operating in the maritime mobile service

# 1	Introduction

Systems and networks operating in the MMS are used for broadband data-links to support various applications such as remote sensing, e.g. earth sciences, land management, energy distribution.

These maritime mobile systems may operate on a 24/7 basis to support security, law enforcement, humanitarian assistance efforts and search and rescue throughout the 4 800-4 990 MHz frequency range in their own territories. Some operations can also take place in international airspace and waters (e.g. to fight against piracy, to support humanitarian operations, to escort ships, for deep sea rescue, for search and rescue/emergency operations at sea, etc).It can be single link involving (AMS and/or) MMS stations or a mesh networks involving several (AMS stations and/or) MMS stations.

# 2	Operational deployment

The maritime mobile system listed in Table 2 uses maritime mobile service data links (MDL) to create a mesh network radio system between ship stations and ground stations to transfer data between nodes. These transmissions could include ship-to-ship, ship-to-coast, or coast-to-ship datalinks. This system can be deployed near a coast or out in international waters.

The usage of this system supports several operations, such as maritime search and rescue, disaster relief, and surveillance. These radio systems are installed on ship stations and ground stations along the coast to allow for datalinks required to transfer data such as imaging and video amongst the users of this mesh network. The mesh network allows for the ships to communicate with other vessels both near port and out in open waters with enough bandwidth capacity to facilitate multiple users over large areas. The links utilized are expected to only extend to radio-line of sight, however there may be multiple nodes in the mesh network and the deployment is expected to cover an area larger (e.g. line-of-sight link) than any one individual desired link.

Some of these operations in the international waters can be planned in advance, whereas some other operations may take place in unpredictable time and location.

# 3	Technical characteristics of systems operating in the maritime mobile service

Typical technical characteristics for representative maritime data links for the frequency range 4 400-4 990 MHz are provided in Table 2.

## 3.1	Transmitter and receiver characteristics

The maritime mobile systems operating or planned to operate within the frequency range 4 400-4 990 MHz typically use digital modulations. A given transmitter may be capable of radiating more than one waveform.

## 3.2	Antenna characteristics

The maritime mobile system listed in Table 2 may use various types of omnidirectional antennas that can be installed on either the ship station or ground station. These omnidirectional antenna gains are typically between 2.5 and 6 dBi.

# 4	Protection criteria

An increase in receiver effective noise of 1 dB would result in significant degradation in communication range.

Such an increase in effective receiver noise level corresponds to an (I + N)/N ratio of 1.26, or an I/N ratio of about −6 dB. This represents the required protection criterion for the MMS systems referenced herein from interference due to another radiocommunication service. If multiple potential interference sources are present, protection of the MMS systems requires that this criterion is not exceeded due to the aggregate interference from the multiple sources.

TABLE 2

Typical technical characteristics of representative systems operating in the maritime mobile service in the frequency range 4 400-4 990 MHz

| Parameter | Units | System 1 Shipborne |  |  | System 1 Ground |  |  | System 2 Shipborne | System 2 Ground |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |  |  |  |  |
| Tuning range | MHz | 4 400-4 940 |  |  | 4 400-4 940 |  |  | 4 800-4 990 | 4 800-4 990 |
| Power output | dBm | 39 |  |  | 39 |  |  | 46 | 46 |
| Bandwidth (3 dB) | MHz | 5.6/11.3/22.6 |  |  | 5.6/11.3/22.6 |  |  | 40/50/60/80/100 (software configurable) | 40/50/60/80/100 (software configurable) |
| Receiver |  |  |  |  |  |  |  |  |  |
| Tuning range | MHz | 4 400-4 940 |  |  | 4 400-4 940 |  |  | 4 800-4 990 | 4 800-4 990 |
| Selectivity (3 dB) | MHz | 5.6/11.3/22.6 |  |  | 5.6/11.3/22.6 |  |  | 40/50/60/80/100 | 40/50/60/80/100 |
| Noise figure | dB | 6 |  |  | 6 |  |  | 5 | 5 |
| Thermal noise level | dBm | -101 to -93 |  |  | -101 to -93 |  |  | −93 … -89 | −93 … -89 |
| Antenna |  |  |  |  |  |  |  |  |  |
| Antenna type |  | Omnidirectional |  |  | Omni-directional |  |  | Directional (steerable, MIMO) | Directional (steerable, MIMO) |
| Antenna gain | dBi | 6 | 4.2 | 2.5 | 6 | 4.2 | 2.5 | 15 | 15 |
| 1st sidelobe | dBi | N/A(1) |  |  | N/A(1) |  |  | N/A(1) | N/A(1) |
| Polarization |  | Vertical |  |  | Vertical |  |  | Vertical | Vertical |
| Antenna pattern |  | N/A(1) |  |  | N/A(1) |  |  | Rec ITU-R F.1336 | Rec ITU-R F.1336 |
| Horizontal beamwidth | Degrees | 360 |  |  | 360 |  |  | 65 | 65 |
| Vertical beamwidth | Degrees | 30 | 37 | 69 | 30 | 37 | 69 | 90 | 90 |
| Notes: (1) N/A – Not applicable. |  |  |  |  |  |  |  |  |  |

______________