ECC Recommendation (YY)XX

Cross-border coordination for Mobile/Fixed Communications Networks (MFCN) in the frequency band 26 GHz

Approved DD Month YYYY

# introduction

This Recommendation covers cross-border coordination scenarios for Wideband (WB) vs. Wideband systems in the 24.25-27.5 GHz (hereinafter 26 GHz) frequency band but does not address cross-border coordination of MFCN vs. other systems.

In this Recommendation, Wideband systems include New Radio (NR) for mobile and fixed wireless access (FWA) deployments.

This Recommendation covers only unsynchronised operation due to envisioned usage of 26 GHz frequency band at the border and administrations/operators may agree on synchronisation of the networks in bilateral/multilateral agreement/arrangement.

# ECC recommendation of YY(XX) on cross-border coordination for Mobile/Fixed Communications Networks (MFCN) in the frequency band 26 GHz

“The European Conference of Postal and Telecommunications Administrations

considering

- that ECC Decision (18)06 [1] provides the harmonised technical conditions for Mobile/Fixed Communications Networks (MFCN) in the 26 GHz frequency band

- that 26 GHz frequency band will mainly be used for urban and suburban hotspot areas but can also be used in rural areas (e.g. industrial and agriculture usage)

- that differences in the market demand for the spectrum for MFCN and different authorisation regimes across CEPT countries is likely to lead to different timescales concerning the introduction of MFCN in the 26 GHz frequency band

- that bilateral/multilateral agreements will be needed concerning the use of MFCN in one country and other systems in a neighbouring country when MFCN is close to the border of the neighbouring country

- that ECC Report 303 [2] provides guidance to administrations for coexistence between 5G systems and fixed links in the 26 GHz frequency band

- that radioastronomy stations are observing in the adjacent (passive) frequency band 23.6-24 GHz and require specific national measures for their protection

- that frequency planning of MFCN in border areas will be based on coordination between national administrations in cooperation with their operators

- that different administrations may wish to adopt different approaches to cross-border coordination

- that administrations may diverge from the technical parameters, propagation models and procedures described in this Recommendation subject to bilateral/multilateral agreements

- that coordination is necessary between neighbouring countries operating different technologies and different channel bandwidths in the same frequency band in border areas

- that in the case of operator arrangements approved by national administrations it is possible to deviate from this Recommendation

- that Physical-Layer Cell Identity (PCI) coordination may be necessary for NR systems to avoid unnecessary signalling load and handover failures

Recommends

- that coordination between MFCN systems in border areas should be based on bilateral/multilateral agreements between administrations

- that coordination between MFCN systems and other systems in neighbouring countries should be based on bilateral/multilateral agreements between administrations

- that operation of MFCN systems should be based on the principles and the field strength levels provided in ANNEX 1:

- that field strength predictions should be made using the appropriate propagation models defined in ANNEX 2:

- that if the field strength levels in ANNEX 1: are exceeded, the coordination is required and system characteristics detailed in ANNEX 3: should be used in the request

- that MFCN systems using NR technology in border areas should use the PCIs, where needed, as provided in ANNEX 4:

- that administrations should encourage and facilitate the establishment of arrangements between operators in different countries with the aim to enhance the efficient use of the spectrum and to optimise the coverage/throughput in their respective border areas

- that this Recommendation should be reviewed within 5 years of its adoption in the light of practical experience of its application and of the operation of MFCN systems

Note:

Please check the Office documentation database https://docdb.cept.org/ for the up to date position on the implementation of this and other ECC Recommendations.

- FIELD STRENGTH LEVELS FOR THE CROSS-BORDER OPERATION between mfcn systems

In this annex, field strength values are given for unsynchronised cross-border scenario between wideband vs wideband MFCN systems.

Field strength values in this annex are median values for base stations using Active Antenna Systems (AAS), i.e. an antenna that consists of an array of active elements.

Administrations/operators may agree on other field strength values, preferential frequency usage and synchronisation based on bilateral/multilateral agreements/arrangements.

AAS base stations of unsynchronised TDD systems on both sides of the borderline in the 26 GHz frequency band 26 GHz for all PCIs (in case of NR) may be used without coordination with a neighbouring country if the mean field strength of each cell produced by the base station does not exceed a value of 62 dBµV/m/(200 MHz) (which corresponds to SSB field strength level of 69 dBµV/m/(120 kHz) for NR) at a height of 3 m above ground level at the borderline between countries.

- PROPAGATION MODELS

The following method is proposed for assessment of anticipated interferences inside neighbouring country based on established field strength levels. Due to complexity of radiowave propagation nature, different methods can be considered by administrations and are included here for guidance purposes only.

It should be noted that following method provides theoretical predictions. It is practically impossible to recreate this method with measurement procedures in the field. Therefore only some approximation of measurements could be used to check compliance with this method based on practical measurement procedures. The details of such approximation are not included in this recommendation and should be negotiated between countries based on their radio monitoring practices.

Path loss model

The basic model to be used for assessment of anticipated field strength level and to decide, if coordination is necessary, is Report ITU-R M.2412 3GPP TR 38.901 [3] (equivalent to Report ITU-R M.2412 [4]). This model includes clutter loss and as such it is not to be combined with other clutter loss models. This model is to be employed using a receiving antenna height hMS = 3 m and Line of Sight (LoS) probability (UMa LoS probability for base station antenna height hBS ≥ 20 m and Umi LoS probability for hBS ˂ 20 m).

Administrations and/or operators concerned may agree to deviate from the aforementioned model by mutual consent.

- EXCHANGE OF INFORMATION

When requesting coordination the relevant characteristics of the base station should be forwarded to the administration affected. All of the following characteristics should be included:

- carrier frequency (GHz)

- name of transmitter station

- country of location of transmitter station

- geographical coordinates (W/E, N; WGS84)

- antenna height (m)

- antenna polarisation

- antenna azimuth (deg)

- directivity in antenna systems or antenna gain (dBi)

- effective radiated power (dBW)

- expected coverage zone

- date of entry into service (month, year)

- PCI (physical-layer cell-identity) numbers used in case of NR

- antenna tilt (deg / electric and mechanic tilt)

- antenna pattern(s) or envelope of data (traffic) channel

- antenna pattern(s) or envelope of control (SSB in case of NR) channel

In addition, when TDD systems are synchronised, the following characteristics should be included as well:

- frame structure, including the special slot “S” configuration (the format at symbol level for slots between downlink and uplink slots)

- time base (start of UTC second epoch)

- Global Synchronisation Channel Number (GSCN) in case of NR

The administration affected shall evaluate the request for coordination and shall within 30 days notify the result of the evaluation to the administration requesting coordination.

During the coordination procedure, an administration may request additional information.

If no reply is received by the administration requesting coordination within 30 days it may send a reminder to the administration affected. An administration not having responded within 30 days following communication of the reminder shall be deemed to have given its consent and the code coordination may be put into use with the characteristics given in the request for coordination.

The periods mentioned above may be extended by common consent.

As a basis during the exchange of information besides listed characteristics above administrations could use formats created within ITU in accordance with Resolution 906 (rev. WRC-15) [7].

- PHYSICAL-LAYER CELL IDENTITIES (PCIs) FOR NR

ETSI TS 136 211 [5] defines 168 “unique physical-layer cell-identity groups” in §6.11, numbered 0..167, hereafter called “PCI groups” for LTE. Within each PCI group there are three separate PCIs giving 504 PCIs in total.

For NR in ETSI TS 138 211 [6] (§7.4.2) the number of physical-layer cell-identity groups (different cell IDs) have been increased to 336, numbered 0..335.

Administrations should apply sharing of PCIs in border areas, an equitable distribution of 1008 PCIs, for preferential and non-preferential PCIs as proposed in the Table 1 below.

Each country should use their own preferential PCIs as a result of sharing of PCIs, depending on cross-border co-ordination scenario and field strength level.

Sharing of PCIs between operators of neighbouring countries should only be applied where synchronisation signal centre frequencies used in the neighbouring countries are aligned independent of the channel bandwidth or where it is not known whether or not the synchronisation signal centre frequencies used in the neighbouring countries are aligned, or where there is no network in operation in the neighbouring country unless otherwise stated in ANNEX 1: or administration agreements / operator arrangements.

The preferential PCIs of a two country PCI sharing should be applied for a base station if the level of field strength relating to non-preferential PCIs could be exceeded at the borderline of only one neighbouring country. The preferential PCIs of a three country PCI sharing should be applied for a base station if the level of field strength related to non-preferential PCIs could be exceeded at the borderline of only two neighbouring countries.

As shown in the Table 1 below, the PCIs for NR are divided into 6 sub-sets containing each one sixth of the available PCIs. Each country is allocated three sets (half of the PCIs) in a bilateral case and two sets (one third of the PCIs) in a trilateral case, therefore dividing the PCI groups or PCIs is equivalent. Four types of countries are defined in such a way that no country will use the same code set as any one of its neighbours. The following lists describe the distribution of European countries:

Type country 1: AZE, BEL, CVA, CYP, CZE, DNK, E, FIN, GRC, IRL, ISL, LTU, MCO, SMR, SRB, SUI, SVN and UKR

Type country 2: AND, BIH, BUL, D, EST, G, GEO, HNG, I and MDA

Type country 3: ALB, AUT, F, HOL, HRV, MLT, POL, POR, ROU and S

Type country 4: LIE, LUX, LVA, MKD, MNE, NOR, SVK and TUR

(Note: Country type map can be found in the figure below).

For each type of country, the following Table 1 and Figure 1 describe the sharing of the PCIs with its neighbouring countries, with the following conventions of writing:

|  | Ppreferential PCI |
| --- | --- |
|  | non-preferential PCI |

Table 1: PCI sub-sets for use in border areas

| PCI | Set A | Set B | Set C | Set D | Set E | Set F |  | PCI | Set A | Set B | Set C | Set D | Set E | Set F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Country 1 | 0..83 504-587 | 84..167 588..671 | 168..251 672..755 | 252..335 756..839 | 336..419 840..923 | 420..503 924..1007 |  | Country 2 | 0..83 504-587 | 84..167 588..671 | 168..251 672..755 | 252..335 756..839 | 336..419 840..923 | 420..503 924..1007 |
| Border 1-2 |  |  |  |  |  |  |  | Border 2-1 |  |  |  |  |  |  |
| Zone 1-2-3 |  |  |  |  |  |  |  | Zone 2-3-1 |  |  |  |  |  |  |
| Border 1-3 |  |  |  |  |  |  |  | Border 2-3 |  |  |  |  |  |  |
| Zone 1-2-4 |  |  |  |  |  |  |  | Zone 2-1-4 |  |  |  |  |  |  |
| Border 1-4 |  |  |  |  |  |  |  | Border 2-4 |  |  |  |  |  |  |
| Zone 1-3-4 |  |  |  |  |  |  |  | Zone 2-3-4 |  |  |  |  |  |  |
| PCI | Set A | Set B | Set C | Set D | Set E | Set F |  | PCI | Set A | Set B | Set C | Set D | Set E | Set F |
| Country 3 | 0..83 504-587 | 84..167 588..671 | 168..251 672..755 | 252..335 756..839 | 336..419 840..923 | 420..503 924..1007 |  | Country 4 | 0..83 504-587 | 84..167 588..671 | 168..251 672..755 | 252..335 756..839 | 336..419 840..923 | 420..503 924..1007 |
| Border 3-2 |  |  |  |  |  |  |  | Border 4-1 |  |  |  |  |  |  |
| Zone 3-1-2 |  |  |  |  |  |  |  | Zone 4-1-2 |  |  |  |  |  |  |
| Border 3-1 |  |  |  |  |  |  |  | Border 4-2 |  |  |  |  |  |  |
| Zone 3-1-4 |  |  |  |  |  |  |  | Zone 4-2-3 |  |  |  |  |  |  |
| Border 3-4 |  |  |  |  |  |  |  | Border 4-3 |  |  |  |  |  |  |
| Zone 3-2-4 |  |  |  |  |  |  |  | Zone 4-3-1 |  |  |  |  |  |  |

Note

- In certain specific cases (e.g. AUT/HRV) where the distance between two countries of the same type number is very small (< few 10s km) and at the same time harmful interference for that distance could occur, it may be necessary to address the situation in bilateral /multilateral coordination agreements as necessary, and further subdivision of the allocated PCIs may be included in certain areas.

- Figure 1: Country type map

- List of References

- ECC Decision (18)06: “Harmonised technical conditions for Mobile/Fixed Communications Networks (MFCN) in the band 24.25-27.5 GHz”, approved 6 July 2018, latest amended 20 November 2020

- ECC Report 303: “Guidance to administrations for coexistence between 5G and fixed links in the 26 GHz ban“ ("Toolbox")”, approved 5 July 2019

- 3GPP TR 38.901 ([Version number], date): “5G; Study on channel model for frequencies from 0.5 to 100 GHz”

- Report ITU-R M.2412-0 (11/2017): “Guidelines for evaluation of radio interface technologies for IMT-2020”

- ETSI TS 136 211 ([Version number], date): “LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation”

- ETSI TS 138 211 ([Version number], date): “5G; NR; Physical channels and modulation”

- Resolution 906 (rev. WRC-15): “Electronic submission of notices for terrestrial services to the Radiocommunication Bureau and exchange of data between administrations”

SSB – Synchronisation Signal Block for 5G NR

For  hBS ≥  20 m UMa LoS probability should be used. For hBS  ˂ 20 m Umi LoS probability should be used.

Uma – Urban Macro

Umi – Urban Micro

e.g. as used by members of the HCM-Agreement Error: Reference source not found