|  |  |  | ECC PT1_CG4G(23)017rev1 |  |
| --- | --- | --- | --- | --- |
| ECC PT1 CG 4 GHz #7 |  |  |  |  |
| Web Meeting, 13 March 2023, 9:30-12:30 |  |  |  |  |
| Date issued: | 12 March 2023 |  |  |  |
| Source: | Lithuania |  |  |  |
| Subject: | Other parameters and assumptions for studies |  |  |  |
| Group membership required to read? (Y/N) |  | N |  |  |
| Summary: |  |  |  |  |
| This input identifies most of interference scenarios under task for studies and proposes the new template to cover propagation characteristics (including clutter and wall loss parameters). |  |  |  |  |
| Proposal: |  |  |  |  |
| invites Group to - consider replacing material in sections 1 and 2 in document ECC PT1_CG4G(23)013_Annex 2 by the proposal in this input contribution; - consider the new structure for scenarios to be used for sections 4 and 5 of draft CEPT Report. |  |  |  |  |
| Background: |  |  |  |  |
| CG 4GHz #6 (1-2 March, Germany, Mainz) document ECC PT1_CG4G(23)013_Annex 2 (Other parameters and assumptions for studies - CG4G_6 output). |  |  |  |  |

TABLE OF CONTENTS

1	Coexistence scenarios	2

2	Propagation characteristics for coexistence scenarios	5

2.1	In-band sharing studies	5

2.1.1	Between WBB LMP in the 3.8-4.2 GHz frequency band	5

2.1.1.1	Co-channel	5

2.1.1.2	Adjacent channel	5

2.1.2	Between WBB LMP and incumbent users in the 3.8-4.2 GHz frequency band	6

2.1.2.1	Between WBB LMP and FS	6

2.1.2.2	Between WBB LMP and FSS (s-E)	6

2.2	Adjacent band compatibility studies	6

2.2.1	WBB LMP in 3.8-4.2 GHz and systems below 3.8 GHz	6

2.2.1.1	Between WBB LMP and MFCN	6

2.2.1.2	Between WBB LMP and FS	8

2.2.1.3	Between WBB LMP and FSS (s-E)	8

2.2.2	WBB LMP in 3.8-4.2 GHz and systems above 4.2 GHz	8

2.2.2.1	Between WBB LMP and WAIC	8

2.2.2.2	Between WBB LMP and RA	8

# Coexistence scenarios

The layout of the scenarios can be illustrated in Figure 1.

Figure 1: Layout of scenarios

Allocation of services and application according to ECO Frequency Information System (EFIS) for frequency range 3400-4400 MHz are provided in Figure 2. Applications to be covered by studies are summarised in Table 1.

Figure 2: Services and applications in ECA Table for frequency range 3400-4400 MHz

Table 1: Services and systems to be considered for studies

| Studies | Allocation | Application |
| --- | --- | --- |
| In-band (sharing): 3800-4200 MHz | FIXED | Fixed link |
|  | FIXED-SATELLITE (space-to-Earth) | Earth station |
|  | MOBILE | WBB LMP |
| Adjacent band (compatibility): 3400-3800 MHz and 4200-4400 MHz, as applicable | FIXED | Fixed link |
|  | FIXED-SATELLITE (space-to-Earth) | Earth station |
|  | MOBILE | MFCN |
|  | AERONAUTICAL MOBILE (R) | WAIC |
|  | AERONAUTICAL RADIONAVIGATION | RA |
| Note: WBB LMP – terrestrial wireless broadband systems providing local-area (i.e. low/medium power) network connectivity; MFCN – mobile/fixed communications networks which includes IMT and other communications networks in the mobile and fixed services" which would include fixed wireless access but not point-to-point links; WAIC – wireless avionics intra-communication; RA – Radio Altimeters. |  |  |

General overview of interference scenarios is provided in Table 2.

Table 2: Interference scenarios (interference links)

| Interfering system | Victim system | Studies | Section |
| --- | --- | --- | --- |
| Between WBB LMP |  |  |  |
| WBB LP | WBB LP | In-band: co-channel, adj-channel | Table 4 (co-channel), Table 6 (adjacent channel) |
| WBB MP | WBB MP | In-band: co-channel, adj-channel |  |
| WBB MP | WBB LP | In-band: co-channel, adj-channel |  |
| Between WBB LMP and MFCN |  |  |  |
| WBB LP | MFCN | Adj-band | Table 9 (WBB LMP into MFCN), Table 11 (MFCN into WBB LMP) |
| WBB MP | MFCN | Adj-band |  |
| MFCN | WBB LP | Adj-band |  |
| MFCN | WBB MP | Adj-band |  |
| Between WBB LMP and FS |  |  |  |
| WBB LP | FS | In-band: co-channel, adj-channel | Table 7 (co-channel and adjacent channel) |
| WBB MP | FS | In-band: co-channel, adj-channel |  |
| WBB LP | FS | Adj-band | Table 12 |
| WBB MP | FS | Adj-band |  |
| Between WBB LMP and FSS (s-E) |  |  |  |
| WBB LP | FSS (s-E) | In-band: co-channel, adj-channel | Table 8 (co-channel and adjacent channel) |
| WBB MP | FSS (s-E) | In-band: co-channel, adj-channel |  |
| WBB LP | FSS (s-E) | Adj-band | Table 13 |
| WBB MP | FSS (s-E) | Adj-band |  |
| Between WBB LMP and WAIC |  |  |  |
| WBB LP | WAIC | Adj-band | Table 14 |
| WBB MP | WAIC | Adj-band |  |
| Between WBB LMP and RA |  |  |  |
| WBB LP | RA | Adj-band | Table 15 |
| WBB MP | RA | Adj-band |  |

Mobile service systems deployment can by defined in the following way:

BS/UE of WBB LMP:

- Low Power (LP) Base Station (BS);

- Outdoors: LP BSout non-AAS

- Indoors: LP BSin non-AAS

- Medium Power (MP) Base Station (BS);

- Outdoors: MP BSout non-AAS

- Outdoors: MP BSout AAS

- User equipment (UE)

- Outdoors: UEout

- Indoors: UEin

BS/UE of MFCN (5G commercial in 3.4-3.8 GHz band):

- Wide Aea (WA) Base Station (BS) – Macrocell

- Outdoors: MFCN BSout_macro AAS

- Medium Range (MR) Base Station (BS) – Microcell

- Outdoors: MFCN BSout_micro non-AAS

- Outdoors: MFCN BSout_micro AAS

- Local Area (LA) Base Station (BS) – Picocell

- Indoors: MFCN BSin_pico non-AAS

- User equipment (UE)

- Outdoors: UEout

- Indoors: UEin

Table 3: Interfering or victim system links

| System | System link | Comment |
| --- | --- | --- |
| WBB LP | BSout  UEout | Outdoor BS link with outdoor UE |
| WBB LP | BSout  UEin | Outdoor BS link with indoor UE |
| WBB LP | BSin  UEin | Indoor BS link with indoors UE |
| WBB MP | BSout  UEout | Outdoor BS link with outdoor UE |
| MFCN | BSout_macro  UEout BSout_micro  UEout BSin_pico  UEin | Outdoor BS link with outdoor UE (Macro cell, Micro cell) Indoor BS link with indoor UE (Pico cell) |
| FS | Tx/Rx  Tx/Rx | Link of fixed link between transmitter and receiver |
| FSS (s-E) | Tx  Rx | Earth station receiving from satellite |
| WAIC | Tx/Rx  Tx/Rx | WAIC link onboard aircraft |
| RA | Tx  Rx | Receiving radio altimeter onboard aircraft |

# Propagation characteristics for coexistence scenarios

## In-band sharing studies

### Between WBB LMP in the 3.8-4.2 GHz frequency band

#### Co-channel

Table 4: Coexistence scenarios between WBB LMP, unsynchronised, co-channel

| Interferer (ILT) |  |  |  | Victim (VLR) |  |  | Propagation model | Antenna height |  | Clutter |  | BEL |  | Environment |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | ILT | VLR | ILT | VLR | ILT | VLR | ILT | VLR |
| LP | BSout | non-AAS |  | LP | BSout | non-AAS |  |  |  |  |  |  |  |  |  |
| LP | BSout | non-AAS |  | LP | BSin | non-AAS |  |  |  |  |  |  |  |  |  |
| MP | BSout | non-AAS |  | LP | BSout | non-AAS |  |  |  |  |  |  |  |  |  |
| MP | BSout | non-AAS |  | LP | BSin | non-AAS |  |  |  |  |  |  |  |  |  |
| MP | BSout | AAS |  | LP | BSout | non-AAS |  |  |  |  |  |  |  |  |  |
| MP | BSout | AAS |  | LP | BSin | non-AAS |  |  |  |  |  |  |  |  |  |
| MP | BSout | non-AAS |  | MP | BSout | non-AAS |  |  |  |  |  |  |  |  |  |
| MP | BSout | non-AAS |  | MP | BSout | AAS |  |  |  |  |  |  |  |  |  |
| MP | BSout | AAS |  | MP | BSout | non-AAS |  |  |  |  |  |  |  |  |  |
| MP | BSout | AAS |  | MP | BSout | AAS |  |  |  |  |  |  |  |  |  |

Table 5: System link of WBB LMP

| Base station |  |  |  |  |  | User equipment |  | Propagation model |  | Antenna height |  |  | Clutter |  |  | BEL |  |  | Environment |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | BS |  | UE | BS |  | UE | BS |  | UE | BS |  | UE |  |
| LP | BSout | non-AAS |  |  | UEout |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LP | BSout | non-AAS |  |  | UEin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LP | BSin | non-AAS |  |  | UEin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MP | BSout | non-AAS |  |  | UEout |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MP | BSout | AAS |  |  | UEout |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### Adjacent channel

Table 6: Coexistence scenarios between WBB LMP, unsynchronised, adjacent channel

[same as Table 4]

### Between WBB LMP and incumbent users in the 3.8-4.2 GHz frequency band

#### Between WBB LMP and FS

Table 7: Coexistence scenarios between WBB LMP and FS in 3.8-4.2 GHz frequency band

#### Between WBB LMP and FSS (s-E)

Table 8: Coexistence scenarios between WBB LMP and FSS (s-E) in 3.8-4.2 GHz frequency band

## Adjacent band compatibility studies

### WBB LMP in 3.8-4.2 GHz and systems below 3.8 GHz

#### Between WBB LMP and MFCN

Table 9: Compatibility scenarios between WBB LMP (interferer) and MFCN (victim) below 3.8 GHz

| Interferer (ILT) |  |  |  | Victim (VLR) |  |  | Propagation model | Antenna height |  | Clutter |  | BEL |  | Environment |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | ILT | VLR | ILT | VLR | ILT | VLR | ILT | VLR |
| LP | BSout | non-AAS |  | MFCN | BSout_macro | AAS |  |  |  |  |  |  |  |  |  |
| LP | BSout | non-AAS |  | MFCN | BSout_micro | non-AAS |  |  |  |  |  |  |  |  |  |
| LP | BSout | non-AAS |  | MFCN | BSout_micro | AAS |  |  |  |  |  |  |  |  |  |
| LP | BSout | non-AAS |  | MFCN | BSin_pico | non-AAS |  |  |  |  |  |  |  |  |  |
| LP | BSin | non-AAS |  | MFCN | BSout_macro | AAS |  |  |  |  |  |  |  |  |  |
| LP | BSin | non-AAS |  | MFCN | BSout_micro | non-AAS |  |  |  |  |  |  |  |  |  |
| LP | BSin | non-AAS |  | MFCN | BSout_micro | AAS |  |  |  |  |  |  |  |  |  |
| LP | BSin | non-AAS |  | MFCN | BSin_pico | non-AAS |  |  |  |  |  |  |  |  |  |
| MP | BSout | non-AAS |  | MFCN | BSout_macro | AAS |  |  |  |  |  |  |  |  |  |
| MP | BSout | non-AAS |  | MFCN | BSout_micro | non-AAS |  |  |  |  |  |  |  |  |  |
| MP | BSout | non-AAS |  | MFCN | BSout_micro | AAS |  |  |  |  |  |  |  |  |  |
| MP | BSout | non-AAS |  | MFCN | BSin_pico | non-AAS |  |  |  |  |  |  |  |  |  |
| MP | BSout | AAS |  | MFCN | BSout_macro | AAS |  |  |  |  |  |  |  |  |  |
| MP | BSout | AAS |  | MFCN | BSout_micro | non-AAS |  |  |  |  |  |  |  |  |  |
| MP | BSout | AAS |  | MFCN | BSout_micro | AAS |  |  |  |  |  |  |  |  |  |
| MP | BSout | AAS |  | MFCN | BSin_pico | non-AAS |  |  |  |  |  |  |  |  |  |

Table 10: System link of MFCN

| Base station |  |  |  | User equipment | Propagation model | Antenna height |  | Clutter |  | BEL |  | Environment |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | BS | UE | BS | UE | BS | UE | BS | UE |
| MFCN | BSout_macro | AAS |  | UEout |  |  |  |  |  |  |  |  |  |
| MFCN | BSout_micro | non-AAS |  | UEout |  |  |  |  |  |  |  |  |  |
| MFCN | BSout_micro | AAS |  | UEout |  |  |  |  |  |  |  |  |  |
| MFCN | BSin_pico | non-AAS |  | UEin |  |  |  |  |  |  |  |  |  |

For system link of WBB LMP see Table 5.

Table 11: Compatibility scenarios between WBB LMP (victim) and MFCN (interferer) below 3.8 GHz

| Interferer (ILT) |  |  |  |  |  | Victim (VLR) |  |  |  | Propagation model |  | Antenna height |  |  | Clutter |  |  | BEL |  |  | Environment |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | ILT |  | VLR | ILT |  | VLR | ILT |  | VLR | ILT |  | VLR |  |
| MFCN | BSout_macro | AAS |  |  | LP |  | BSout | non-AAS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MFCN | BSout_micro | non-AAS |  |  | LP |  | BSout | non-AAS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MFCN | BSout_micro | AAS |  |  | LP |  | BSout | non-AAS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MFCN | BSin_pico | non-AAS |  |  | LP |  | BSout | non-AAS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MFCN | BSout_macro | AAS |  |  | LP |  | BSin | non-AAS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MFCN | BSout_micro | non-AAS |  |  | LP |  | BSin | non-AAS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MFCN | BSout_micro | AAS |  |  | LP |  | BSin | non-AAS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MFCN | BSin_pico | non-AAS |  |  | LP |  | BSin | non-AAS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MFCN | BSout_macro | AAS |  |  | MP |  | BSout | non-AAS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MFCN | BSout_micro | non-AAS |  |  | MP |  | BSout | non-AAS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MFCN | BSout_micro | AAS |  |  | MP |  | BSout | non-AAS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MFCN | BSin_pico | non-AAS |  |  | MP |  | BSout | non-AAS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MFCN | BSout_macro | AAS |  |  | MP |  | BSout | AAS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MFCN | BSout_micro | non-AAS |  |  | MP |  | BSout | AAS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MFCN | BSout_micro | AAS |  |  | MP |  | BSout | AAS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MFCN | BSin_pico | non-AAS |  |  | MP |  | BSout | AAS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

For system link of WBB LMP see Table 5 and for MFCN see Table 10.

#### Between WBB LMP and FS

Table 12: Compatibility scenarios between WBB LMP and FS below 3.8 GHz

#### Between WBB LMP and FSS (s-E)

Table 13: Compatibility scenarios between WBB LMP and FSS (s-E) below 3.8 GHz

### WBB LMP in 3.8-4.2 GHz and systems above 4.2 GHz

#### Between WBB LMP and WAIC

Table 14: Compatibility scenarios between WBB LMP and WAIC above 4.2 GHz

#### Between WBB LMP and RA

Table 15: Compatibility scenarios between WBB LMP and RA above 4.2 GHz