# Introduction

This sensitivity sharing study analyses the interference scenario between WBB LMP transmitters and Fixed Satellite Services (FSS) receivers in the band 3800-4200 MHz by simulating examples of two existing FSS earth stations (ES) in Germany.

These Minimum Coupling Loss (MCL) simulation results are analysed to determine needed conditions for their coexistence while ensuring the protection of the FSS ES. For this, the interference from the WBB LMP base stations (BS) is studied, not vice versa. Interference from FSS ES into WBB LMP BS or user equipment (UE) is not considered in this study.

For the MathWorks MATLAB simulations and plots, terrain data from the German “Federal Agency for Cartography and Geodesy” (BKG) is used, also the CLC (“CORINE Land Cover”) data from the “European Environment Agency” (EEA) to estimate potential clutter.

This sensitivity study is a complement to the theoretical approaches by applying the agreed WBB LMP BS parameters (see 2.2) to realistic ES scenarios (see 2.1). The ES which are taken into account are existing use cases. Therefore, the theoretically developed parameters will be more tangible.

# INterference Scenario and Key Parameters

In the band 3800-4200 MHz licenced FSS ES are used to communicate with geostationary satellites (GSO).

The rural medium power WBB (MP WBB) BS outdoor/outdoor scenario must be seen as the worst case for FSS ES, as the antenna height for both services is considered above the average clutter height. The transmitted power of the WBB MP BS (PBS,MP) is significantly higher than the transmitted power of a low power WBB (LP WBB) BS (PBS,LP) which leads to higher interference for the WBB MP BS cases.

Still other scenarios such as the WBB LP BS indoor/outdoor scenario are also important to estimate whether small cell deployment would be feasible in this band.

In general, this study concentrates on the WBB LMP BS outdoor/outdoor scenarios, but also includes an indoor/outdoor example.

## Fixed Satelite Service Parameters

Both examples of FSS ES studied (see Table 1) are stationed in predominantly rural areas. The ES data are taken from national assignment allocations and the antennas modelled with Recommendation ITU-R S.580-6 in MATLAB.

Table 1: Real deployment parameters of FSS ES in Germany: Fuchsstadt and DLR

| Parameter | Fuchsstadt | DLR |
| --- | --- | --- |
| Coordinates longitude | 9.9242° | 11.2811° |
| Coordinates latitude | 50.1194° | 48.0861° |
| Antenna size (diameter) | 32 m | 4.8 m |
| Antenna centre height above ground | 20 m | 14 m |
| Maximum antenna gain | 61 dBi | 44 dBi |
| Maximum antenna side lobe gain | 5.9 dBi | -1.2 dBi |
| Minimum elevation | 8.4° | 16.1° |
| Minimum azimuth | 112° | 121° |
| Maximum azimuth | 249° | 240° |
| Receiver system noise temperature TFSS | 50°K | 23°K |

The ES in Fuchsstadt has a large number of different antennas in the studied band with azimuths ranging from 112° to 249°and a minimum elevation angle of 8.4°. With some large sized and high positioned antennas (e.g. 42 m height above ground), ES Fuchsstadt can be considered as one of the ES in Germany with the highest potential for interference by WBB LMP BS. The analysis (see 3.1) looks at the south-orientating main lobes of the antennas, as shown in Table 1.

The DLR (“Deutsches Zentrum für Luft- und Raumfahrt”, German Aerospace Centre) antenna is chosen because it is located in an industrial zone surrounded by rural area. The analysis shows that even a small industrial zone can lead to totally different interference results depending on the clutter assumed (see 3.2).

## WBB LMP and additional Parameters for 3800-4200 MHz

Parameters of the WBB LMP providing local area network connectivity in 3800-4200 MHz are based on the UK/NOR approach (the detailed WBB LMP parameters are listed in ANNEX 1:).

Table 2: Additional parameters for WBB LMP BS

| Parameter | Value | Comment |
| --- | --- | --- |
| f | 3800 MHz | Simulated worst case frequency |
| hBS,LP | 10 m | Base station height low power BS scenario |
| PBS,LP | 11 dBm/MHz | Tx EIRP (for AAS & non-AAS) for low power BS scenario, =18 dBm / 5 MHz for carriers > 20 MHz |
| hBS,MP | 25 m | Base station height medium power BS scenario |
| PBS,MP | 29 dBm/MHz | Tx EIRP (for AAS & non-AAS) for medium power BS scenario, = 36 dBm / 5 MHz for carriers > 20 MHz |

Simulations with SEAMCAT-5.4.1 using the ”Test Propagation Model Tool” for Recommendation ITU-R P.452-16 (with Recommendation ITU-R P.2108-1 for clutter, applied one-sided, rural) showed similar resulting path losses for different frequencies in the 3800-4200 MHz range. It is therefore decided to use 3800 MHz as the worst case frequency for all scenarios.

The BS antenna is assumed to be omnidirectional. Note that a higher antenna gain could result in higher separation distances.

From the experience made with the allocations in the C-Band it is not expected to have 5G channels with a lower bandwidth than 20 MHz. Therefore only WBB power limits for bandwidths wider than 20 MHz are studied.

## FSS protection criteria, scenarios and other assumptions

The study is based on MATLAB simulations using ITU-R P.452-17 and terrain data (DTM) from the BKG (no clutter heights used along the path) in combination with ITU-R P.2108-1 section 3.1 (named “representative clutter section” in this study) to provide more realistic path loss (PL) results than the same Recommendations using section 3.2 (named “statistical clutter section” in this study) instead.

Table 3: Simulation scenarios for WBB LMP into FSS ES

| Scenario No. | FSS ES | WBB BS type | Scenario type | ITU-R P.2108-1 section and scenario comments |
| --- | --- | --- | --- | --- |
| 1a | Fuchsstadt | LP | Outdoor/outdoor, long term protection criterion | Section 3.1 - representative clutter |
| 1b |  |  |  | Section 3.2- statistical clutter, clutter applied one-sided |
| 2a |  | MP | Outdoor/outdoor, long term protection criterion | Section 3.1 - representative clutter |
| 2b |  |  |  | Section 3.2- statistical clutter, no clutter applied |
| 3a |  | LP | Indoor/outdoor, long term protection criterion | Section 3.1 - representative clutter |
| 3b |  |  |  | Section 3.2- statistical clutter, clutter and building entry loss applied one-sided |
| 4 |  | LP | Outdoor/outdoor, short term protection criterion | Section 3.1 - representative clutter |
|  |  | MP |  |  |
| 5a | DLR | LP | Outdoor/outdoor, long term protection criterion | Section 3.1 - representative clutter |
| 5b |  |  |  | Section 3.2- statistical clutter, clutter applied on both sides |
| 5c |  |  |  | Section 3.2- statistical clutter, clutter applied one-sided |
| 6a |  | MP |  | Section 3.1 - representative clutter |
| 6b |  |  |  | Section 3.2- statistical clutter, clutter applied one-sided |
| 6c |  |  |  | Section 3.2- statistical clutter, no clutter applied |

The results for both sections of ITU-R P.2108-1 are included to show their differences in combination with real deployment data (see Table 3).

It should be noted that the representative clutter section of ITU-R P.2108-1 is only applicable up to 3000 MHz, but since there is no other Recommendation dealing with representative clutter above 3000 MHz until now, it is common to use the representative clutter section of ITU-R P.2108-1 for higher frequencies.

In the statistical clutter section of ITU-R P.2108-1, clutter is applied on the WBB LP BS side for the WBB LP BS scenarios, as the maximum antenna height is limited to 10 m. 50 percentage of locations is used to calculate the average clutter loss  (LClutter).

LClutter = 31 dB

No clutter is applied for the WBB MP BS statistical clutter scenarios, as the antenna height is considered above the average clutter height.

For the cases 5a/b/c and 6a/b/c (DLR), clutter application is described in 3.2 of this study.

For the indoor/outdoor WBB LP BS to FSS ES scenarios an additional building entry loss LBEL= 12 dB is applied.

Non-time variant assumptions are made, e.g. both victim services and interfering services are static. Therefore the percentage of time assumed for ITU-R P.452-17 is the percentage of time linked to the protection criteria of the victim service FSS ES.

The long-term protection criterion used is I/NLT = -10.5 dB with a time percentage of pt = 20%.

The short-term protection criterion used is I/NST= -1.3 dB with a time percentage of pt = 0.005%.

The maximum allowed interference level Imax in the FSS receiver is calculated based on the receiver noise temperature (see Table 1).

Imax, LT   = N + I/NLT

Imax, ST = N + I/NST

# Simulations and conclusions

## Fuchsstadt

### Scenario No.1a/b: WBB LP BS, long term protection criterion, outdoor/outdoor

Figure 1: Application of representative clutter (1a, left) and statistical clutter (1b, right)

The results highlight that the application of a fixed statistical attenuation due to clutter significantly reduces the required separation distance.

Here a reduction of the maximum separation distance from 9.3 km (left) to 4.7 km (right) is seen, almost half the distance compared to representative clutter applied. This could lead to potential interference in the ES receiver.

### Scenario No.2a/b: WBB MP BS, long term protection criterion, outdoor/outdoor

Figure 2: Application of representative clutter (2a, left) and statistical clutter (2b, right)

As the MP WBB BS antenna is always above the average clutter height, there is no difference between both approaches. Both cases assume a clutter attenuation of 0 dB on both ends of the propagation path.

The maximum required separation distances go up to 26.5 km.

### Scenario No.3a/b: WBB LP BS, long term protection criterion, indoor/outdoor

Figure 3: Application of representative clutter (3a, left) and statistical clutter (3b, right)

For both indoor/outdoor LP WBB BS to FSS ES scenarios an additional building entry loss LBEL= 12 dB is applied.

The results of the indoor/outdoor scenarios can be compared to the results of the WBB LP BS outdoor/outdoor scenarios in 3.1.1.

The addition of 12 dB building entry loss reduces the necessary protection distance to 8.6 km (left) instead of 9.3 km and to 2.3 km (right) instead of 4.7 km.

The maximum required separation distance in the statistical clutter case is 3.7 times higher compared to the representative clutter case. This could lead to potential interference in the ES receiver.

### Scenario No. 4a/b: WBB LMP BS, short term protection criterion, outdoor/outdoor

Figure 4: Resulting path losses for long term and short term protection criteria for generic cases with different antenna heights (Fuchsstadt)

Figure 4 shows path loss simulations of four generic cases of Fuchsstadt. For this, SEAMCAT-5.4.1: ”Test Propagation Model Tool” is used.

The figure shows path losses for the ES in Fuchsstadt interfered by

low power WBB BS with

long term protection criterion (red),

short term protection criterion (blue),

medium power WBB BS with

long term protection criterion (green),

short term protection criterion (yellow).

No terrain data is used (flat earth) for the generic cases, but the same WBB LMP BS and ES antenna parameters are chosen as before.

Figure 5: WBB LP BS (4a, left) and WBB MP BS (4b, right) with representative clutter

For LP WBB BS both, the short-term protection criterion and the long-term protection criterion, applied to the representative clutter scenario result in maximum required separation distances of up to 9.3 km. This is reasonable, as the short-term propagation effects dominate for path distances where neither the LOS (line of sight) propagation nor the diffraction propagation is dominant anymore (see Figure 4).

For MP WBB BS, the usage of the short-term protection criterion increases the required separation distance up to 35.0 km compared to 26.5 km with the long term protection criterion used. The MP WBB BS antenna is always above the average clutter height. Both cases assume a clutter attenuation of 0 dB on both ends of the propagation path.

Results indicate that real terrain data have a significant impact on the needed separation distance.

Note that only these two cases for real deployment in combination with the short-term protection criterion are studied here.

## DLR

The impact of different clutter assumptions (e.g. representative or statistical clutter and which area is assumed) is seen in Figure 6 and Figure 7 for WBB LP BS and in Figure 8 and Figure 9 for WBB MP BS scenarios.

The study analyses the DLR ES antenna because it is located in an industrial zone surrounded by rural area. This small industrial zone can lead to a totally different resulting protection distance depending on the area assumptions.

### Scenario No.5a/b/c: WBB LP BS, long term protection criterion, outdoor/outdoor

Figure 6: Application of representative clutter (5a, with ES in industrial zone)

The used real terrain data from the CLC defines the ES side as an “industrial zone”, which means the ES antenna height (14 m) is below the average clutter height of 20 m, as defined in TABLE 4 (“Nominal clutter heights and distances”) of ITU-R P.452-17.

Figure 7: Application of statistical clutter on both sides (5b, left) and one-sided (5b, right)

In the statistical clutter section,  LClutter = 31 dB must be applied on both sides, on the WBB LP BS side and on the ES side (“industrial zone”), as both sides are below the average clutter height.

The results highlight, that the application of a fixed statistical attenuation due to clutter significantly reduces the required separation distance. Here a reduction of the maximum separation distance from 17.2 km (Figure 6) to 388 m (Figure 7, left) for clutter on both sides is seen.

Figure 7, on the right, shows the results assuming “rural area” around the receiver instead (hence one-sided clutter on the WBB LP BS side).

### Scenario No.6a/b/c: WBB MP BS, long term protection criterion, outdoor/outdoor

Figure 8: Application of representative clutter (6a, with ES in industrial zone)

The used real terrain data from the CLC defines the ES side as an “industrial zone”, which means the ES antenna height (14 m) is below the average clutter height of 20 m, as defined in TABLE 4 (“Nominal clutter heights and distances”) of ITU-R P.452-17.

Figure 9: Application of one-sided (6b, left) and no statistical clutter (6c, right)

In the statistical clutter section,  LClutter = 31 dB must be applied one-sided, as FSS ES side is below the average clutter height.

The results highlight, that the application of a fixed statistical attenuation due to clutter significantly reduces the required separation distance. Compared to the representative clutter section results (see Figure 8), less, but still high interference from WBB MP BS in the ES is seen.

Figure 9, on the right, shows the results assuming “rural area” around the receiver instead (hence no clutter applied). The maximum separation distances would rise from 17.2 km (see Figure 9, left) to more than 50 km (see Figure 9, right).

## General Conclusions

The simulation results in this sharing study show that the separation distances required to protect FSS ES from WBB LMP BS go up to over 70 km for WBB MP BS and up to 17 km for WBB LP BS when simulated with real terrain data in combination with ITU-R P.2108-1 section 3.1 (representative clutter).

If ITU-R P.2108-1 section 3.2 (statistical clutter) is used instead, the same scenarios result in significantly lower separation distances for most cases.

The different sensitivity simulations show that it is highly dependent on the local terrain data and the antenna heights, how much the WBB LMP BS is interfering the FSS ES.

As the assumed clutter loss in this frequency band can be up to 31 dB for 3800 MHz, the required separation distances will change, whether the WBB LMP BS and/or the FSS ES is/are positioned above or below the average clutter height.

Working also with ITU-R P.2108-1 section 3.1 (representative clutter) to see more realistic results and applying it over 3000 MHz could be one approach.

In conclusion, realistic assumptions must be made to ensure the protection of existing FSS ES from the WBB LMP BS, which cannot be achieved by simply relying on the application of overly optimistic statistical clutter.

WBB LMP PARAMETERS

Table 4: Parameters of the WBB LMP providing local area network connectivity in 3.8-4.2 GHz from UK/NOR approach

| Parameter | Low Power BS | Medium Power BS |
| --- | --- | --- |
| Bandwidth | 10 MHz to 100 MHz | 10 MHz to 100 MHz |
| Antenna height | Outdoor: Limited to a maximum of 10 m above ground Indoor: Any height within building | No limit |
| Deployment scenario | Outdoor/indoor or Indoor-only | Rural areas only |
| BS Tx EIRP limit (for AAS & non-AAS) | 24 dBm / carrier for carriers ≤ 20 MHz; or 18 dBm / 5 MHz for carriers > 20 MHz | 42 dBm / carrier for carriers ≤ 20 MHz; or 36 dBm / 5 MHz for carriers > 20 MHz |
| Maximum terminal power | Mobile/nomadic: TRP 28 dBm (Note 2) Fixed: EIRP 28 dBm (Note 2) | Mobile/nomadic: TRP 28 dBm (Note 2) Fixed: TRP 28 dBm and EIRP 35 dBm/5 MHz (Note 1) |
| Note 1: Higher EIRP limit for fixed terminals in the medium power BS case is to account for the use case of Fixed Wireless Access (FWA) Note 2: The authorisation of 28 dBm includes a 2 dB tolerance consistent with the European harmonisation. |  |  |

Protection criteria for FSS (in-band)

Table 5: Protection criteria for FSS (in-band)

| Frequency Ranges |  | Percentage of time for which the I/N value could be exceeded (%) | I/N Criteria (dB) |
| --- | --- | --- | --- |
| 3 800-4 200 MHz | Long term Short term | 20% 0.005% (NOTE) | −10.5 −1.3 (NOTE) |

NOTE: Studies using these short-term protection criteria could be assessed on the basis that these values were put forward by WP 4A to facilitate and complete the work for WRC-23 agenda items and these values may evolve in the future based on inputs to the ITU-R. WP 4A has not completed its work in developing short-term protection criteria, however WP 5D is invited to consider these short-term protection criteria to the extent practicable.

Propagation Models

Table 6: Propagation models used in the simulations with systems other than MFCN

| Link | Model |
| --- | --- |
| Outdoor LMP BS to FS/FSS receiving earth station | ITU-R P.452-16/ITU-R P.2001-4 (Note 1) |
| Indoor LMP BS to FS/FSS receiving earth station | ITU-R P.452-16/ ITU-R P.2001-4 (Note 1) + Wall Loss 12 dB at each indoor BS or fixed value taken from P.2109 for incremental study (Note 2) |
| Note 1: If the study assumes non-time variant assumptions, e.g. both victim services and interfering services are static, the percentage of time assumed for P.452-16/ ITU-R P.2001-4 should be the percentage of time linked to the protection criteria of the victim service. In the case of a time-varying Monte Carlo analysis, the percentage of time should be random at each iteration along with time variant variables Non time variant variables shall not be randomised. To extend P.452 model time percentage (Tpc) range to 0-100%, the SG3 guidance (or similar) in Liaison statement to WP6A ([198]) should be included, namely, that Tpc range should be 0-100% and for Tpc > 50% the losses are equal to the case Tpc = 50 Where the antenna heights of the transmitter and/or receiver are below the nominal clutter heights specified in ITU-R P.452 Table 4, clutter attenuation based on ITU-R P.452 or ITU-R P.2108 at a specified % shall be considered. In case of ITU-R P.2108 implementation the choice of % should be documented. Note 2: In case of ITU-R P.2109 implementation the choice of value should be documented. |  |

The acronym WBB LMP refers to “terrestrial wireless broadband systems providing local-area (i.e. low/medium power) network connectivity” as defined by the EC mandate.

Newer version of the Recommendation ITU-R P.452-16 which was selected as a possible propagation model (see Table 6)