# Introduction

Different studies addressing IMT and broadband wireless systems have been performed during the previous WRC cycles and results of these analyses can be found in different reports such as:

- Report ITU-R M.2109-0 ‒ Sharing studies between IMT-Advanced systems and geostationary satellite networks in the fixed-satellite service in the 3 400-4 200 and 4 500-4 800 MHz frequency bands,

- Report ITU-R S.2199 ‒ Studies on compatibility of broadband wireless access systems and fixed-satellite service networks in the 3 400-4 200 MHz band and

- Report ITU-R S.2368-0- Sharing studies between International Mobile Telecommunication-Advanced systems and geostationary satellite networks in the fixed-satellite service in the 3 400-4 200 MHz and 4 500-4 800 MHz frequency bands in the WRC study cycle leading to WRC-15.

This study analyses further the sharing and compatibility scenarios between WBB LMP and FSS in the band 3800-4200 MHz. Parameters used for the systems are provided, and the interference scenarios as well as the various assumptions considered, which may differ from those of the previous studies mentioned, are discussed. The results obtained show the separation distance required to limit the interference of WBB LMP systems into FSS earth station reception. Different configurations are also assessed, and their impact into the results is analysed. Finally, proposals are made to define a framework for the coexistence of the two systems in the same or adjacent frequency band.

# INterference Scenario

## Fixed SATELLITE Service Parameters

Parameters for the Fixed Satellite Service in the 3.8 - 4.2 GHz provided in report ITU-R S.2368-0. In that FSS downlink frequency band, receiving FSS earth stations can generally be grouped into four categories: earth stations deployed ubiquitously and/or without individual licensing or registration; individually licensed earth stations; telemetry earth stations; and earth stations that are feeder links for mobile-satellite systems.

The use of the band for satellite communications since the end of the 70’s, the maturity and accessibility of the equipment and the physical characteristics leading to advantageous propagation mechanisms make the band of primary importance for various applications and customer services including very small aperture terminal (VSAT) networks, internet providers, point-to-multipoint links, satellite news gathering, TV and data broadcasting to satellite master antenna television (SMATV), direct-to-home (DTH) receivers, and disaster relief. In particular, it is important to note that many countries receive only earth stations or VSAT terminals are not individually licensed and their number, what makes it typically difficult to know exactly their characteristics or their detailed location.

Description on the use of the band and the characteristics of the receiving FSS earth stations are provided in the document ITU-R S.2368 and further details can be found in ITU-R SM.2109-0.

## FSS PARAMETERS IN THE FREQUENCY Range 3.8-4.2 GHz

FSS C-band systems have been used for decades. These systems are very well known, and there is no major evolution in their characteristics compared to what has been used is the previous studies.

The following table provides parameters that can generally be used for FSS systems operating in the frequency range 3.8-4.2GHz to be considered to conduct sharing analysis with WBB LMP.

Table 1: Typical space-to-Earth FSS parameters in the 3.8-4.2GHz frequency bands

| Parameter | Low Power BS |
| --- | --- |
| Antenna diameters (m) | 1.2, 1.8, 2.5, 3.0, 4.5, 8, 16, 32 |
| Antenna reference pattern | Recommendation ITU-R S.465 |
| Range of Bandwidth | 40 kHz – 72 MHz, 40 MHz |
| Receiving system noise temperature | 100 K for small antennas (1.2-3 m) 70 K for large antennas (4.5 metres and above) |
| Earth Station antenna gain | 43.3 dBi |
| Earth station deployment | All regions, in all locations (sub-urban, semi-urban, urban) (Note 1) |
| Antenna elevation | 10° is considered as the minimum operational elevation angle (Note 2) |
| Power flux-density at the Earth’s surface produced by emissions from a space station | In accordance with RR No. 21.16, Table 21-4 |
| Antenna height (outdoor only) | Rooftop: 15-25 m above ground Ground mounted: 5-10m |
| Note 1: FSS antennas in this band may be deployed in a variety of environments. Smaller antennas (1.8-3.8 metres) are commonly deployed on the roofs of buildings or on the ground in urban, semi-urban or sub-urban locations, whereas larger antennas are typically mounted on the ground and deployed in semi-urban or sub-urban locations. Note 2: 5 degree elevation angle has been considered for sensitivity analysis purpose. |  |

## WBB LMP Parameters for 3.8-4.2 GHz

Parameters for the WBB LMP 3.8-4.2 GHz are based on the UK/NOR approach.

## Propagation model

ITU-R P.452-16 is used to calculate Pathloss (PL) between the FSS earth station and WBB LMP with a time percentage of 20%, to match the FS long-term protection criteria. The study relies on non-time variant assumptions, e.g. both victim services and interfering services are static.

Clutter was applied on the WBB_LP side for the low power BS scenario, as the maximum antenna height is limited to 10m. ITU-R P.2108, with 50% percentage of locations was used to calculate the average clutter loss in urban environment and 30% in sub-urban environment. The associated fixed values used are

LClutter = 31 dB (urban)	 and 		LClutter = 29 dB (sub-urban)

## fss protection criteria

Definition

For the interference of WBB LMP systems to FSS, two interference criteria are considered for use when assessing the interference mechanisms for both “In-band emissions” and “Adjacent band emission (out-of-band or spurious emission)”.

Long-term interference criterion

The long-term interference criterion considered for this analysis is based on Recommendation ITU-R S.1432-1 and used in Report ITU-R S.2368-0. When assessing the interference mechanisms for in-band sharing studies and interference analysis from WBB LMP to FSS, the long-term interference criterion used is expressed

|  | IN= -10.5 dB∆TT=10% | eq. 1 |
| --- | --- | --- |

This value corresponds to the aggregate interference from all co-primary allocation for 20% of any month.

When addressing the adjacent band sharing studies (out-of-band or spurious emission), the aggregate interference from all other sources of interference, for 100% of the time is expressed

|  | IN= -20 dB∆TT=1% | eq. 2 |
| --- | --- | --- |

Short-term interference criterion

Based on Recommendation ITU-R SF.1006, the single entry short-term interference criterion is defined as

|  | IN= -1.3 dB | eq. 3 |
| --- | --- | --- |

that must not be exceeded more than 0.005% of time for a single entry interference.

Calculations

In the expressions of I/N above, the value of N correspond to the clear-sky satellite system noise power as described in Recommendation ITU-R S.1432, which is expressed as

|  | N=10*log10K*T*B | eq. 4 |
| --- | --- | --- |

Where:

K: Boltzmann constant (1.38*10^-23)

T: System temperature (Kelvin)

B: earth station carrier bandwidth (Hz)

We focus our analysis on the noise power at the receiver antenna, so the receiver noise figure is ignored. Using the values provided in the tables above and considering a system bandwidth of B=40 MHz with system temperature T=70 K for an antenna diameter of 4.5m used for the analysis, the value of the system noise is  N = -134.13 dBW

The resulting maximum allowed long-term interference level at the FSS ES receiver antenna is:

| ImaxLT = N + I/N = -134.13 – 10.5 = -144.63 dBW/40MHz = -114.63 dBm/40MHz | eq. 5 |
| --- | --- |

The resulting maximum allowed short-term interference level at the FSS ES receiver antenna is:

| ImaxST = N + I/N = -134.13 - 1.3 = -135.43 dBW/40MHz = -105.43 dBm/40MHz | eq. 6 |
| --- | --- |

# SIngle entry Static interference calculation

We assess first the potential interference from a single WBB LMP interferer positioned around a victim FSS earth station, to see what the limits are in such configuration. Our approach is based on a conventional static analysis that will provide an initial understanding of the possibilities and guide towards an ultimate analysis that would takes into account the interference from the network, using the latest improvements in mobile technology.

## general formula

The interference from transmitting WBB LMP base station received at the victim FSS earth station can be calculated using the equation below:

|  | IWBB_S= PWBB+GWBB+GESφ-Lf,d-LClutter-FDR∆f | eq. 7 |
| --- | --- | --- |

Where:

IWBB_S:		Received interference power level in 1 MHz bandwidth at the earth station receiver caused by the transmission from one single station of the WBB LMP system (dBm)

PWBB:		WBB system transmission power in 1 MHz bandwidth (dBm)

GWBB:		WBB system antenna gain (dB)

φ:		FSS ES antenna elevation angle

GES(φ):	FSS ES reception antenna gain (dB) at elevation angle φ

L(f,d):		Path loss (dB) at frequency (f) and distance (d)

LClutter:		Clutter loss (dB) between transmitter and receiver.

When only out-of-band or spurious emissions are considered and adjacent band interference assessed, the additional parameter is also used:

FDR(Δf):	Frequency dependent rejection (dB) at frequency offset Δf (Hz).

When conducting co-channel interference, a value FDR=0 dB is used, otherwise in case of adjacent-band interference, the corresponding value is calculated from a known formula or it is derived from the ACLR of the WBB LMP transmitter.

## impact of the different parts of the chain

## Attenuation required

As a first approach, we assess the required attenuation from transmitter in order not to exceed the protection criterion of the FSS receiver. This required attenuation can be expressed as:

|  | LMindrequired= PWBB+GWBB+GESφ-IMAX | eq. 8 |
| --- | --- | --- |

Assuming simple assumptions, i.e. antennas pointing each towards the other with a minimum elevation of   φ=5° of the FSS earth station and the free space model channel, the additional attenuation required above the free space loss is expressed (FSL = Free Space Loss):

|  | Ladddrequired= LMindrequired-FSLf,d | eq. 9 |
| --- | --- | --- |

The two figures below shows the attenuation provided by the Free Space Loss (FSL) model and the corresponding attenuation to be added on top of FSL to meet the FSS ES long term protection criterion:

| a) | b) |
| --- | --- |
| - c) | d) |

Figure 1: Free Space model attenuation and additional signal attenuation required

It should be noted that with the considered antenna pattern, there is no impact of the maximum FSS ES antenna gain, as for all diameters the antennas have the same off-axis gain above a minimum elevation angle of 5°.

Also, the required attenuation has been calculated with a WBB medium power with transmit power of PTx =35dBm for a bandwidth of 40 MHz. Reducing or increasing the power of X dB will lead to exactly the same value of the reduction or increase of the required additional attenuation.

The additional attenuation required on top of the FSL can be met by combining different parameters that bring their own contribution to the overall received interference.

## Antenna misalignment

Antenna pointing is one element that can contribute to meet the required attenuation on top of the free space loss. Plotting the antenna gains at transmitter and receiver sides we can see the margin than can be gained when antennas are not pointing each towards the other.

The figures below show the possibilities in terms of margin provided by the FSS earth station antenna pattern, depending on the elevation towards the satellite, which corresponds also to the off-axis from the horizontal plane.

| a) S465 antenna pattern for off-axis of 5° and more | b) Required additional attenuation per FSS elevation |
| --- | --- |

Figure 2: S465 antenna pattern and impact of FSS ES elevation

The offset angle between WBB LMP BS and FSS ES will impact the required additional attenuation. A change from 5° elevation to 20° provides a 15dB margin leading to the same margin on top of FSL and a change up to 48° provides the maximum 24.5dB additional attenuation to FSL compared to 5° elevation, as it can be noticed that there is no further improvement that can be expected from the FSS antenna sidelobes for ES elevations higher than 48°.

In order to appreciate the elevation angle to consider within Europe, the figures below present the elevation contours across Europe, i.e. the elevation angle for FSS ES at different locations across the continent, for a GSO satellite located, e.g. at 19.2E. Case study will be considered for FSS ES located in Luxembourg and in the Netherlands. The locations and corresponding elevation of these two cases are also depicted.

Figure 3: Possible elevation contours for FSS ES at different locations across Europe

The figures show that locations at the extreme south of Europe (Spain, Italy, Greece) are likely to be at elevation between 40° and 45°, while those at the extreme north (Norway, Finland) are between 10° and 15°. The FSS ES site in Luxembourg is at elevation of about 33° and the ES in the Netherlands is at elevation of about 30° towards the GSO satellite.

On the mobile side, the antenna misalignment would also provide some additional attenuation to the path. The figures below present the margin offered by the antenna gain of the WBB LMP antenna for a sectoral peak configuration of model F.1336_5 (more favourable and discriminating configuration than sectoral average).

If only 6 dB attenuation can be obtained from a 47° misalignment in azimuth, the vertical off-axis provided a much better discrimination scheme with 6 dB from the assumption of 6° mechanical down-tilt, 12 dB if the off-axis from the FSS earth station is of 20°, and 15 dB attenuation for an off-axis of 41°.

Figure 4: F.1336-5 antenna pattern and off-axis margin

As it can be observed, this pattern is considering an antenna gain of 10 dBi. The front-to-back attenuation in azimuth (hozitontal plane) is of about 23.7 dB which is equivalent to one sub-urban clutter.

## Channel model

The channel model considered may also play a role in the corresponding level of interference calculated. If the Free Space model may provide a first hint for appreciating the potential impact of WBB LMP BS into FSS ES, a more accurate model would certainly provide more realistic assumptions and results. When comparing the model P.452-16 used for this study with a percentage of time of 20% to the FSL model (see figure below which represents the attenuation provided by P452 at the latitude of the FSS site of Betzdorf), it can be noticed that the two model provide same attenuation until a breaking point at about 15 km where the P452 model starts deviating and gives higher path loss leading to a difference of 40 dB when the distance between the two stations is 50 km. Before the breaking point, there is a small deviation zone where P452 has slightly lower attenuation than free space, which may also impact a little the results from 6 km to the breaking point.

It should be noticed that no terrain model nor obstacle are considered between the antennas, and that the P452 curve and the spotlighted elements (breaking point, deviation zone, gap at 50 km…), may differ depending on the coordinates of the stations, the frequency and other parameters used in the P452 model.

However, this curve demonstrates that the Free Space Model can be used to consider analysis for short distances around the victim station and to define the coordination distance around an FSS ES.

Figure 5: Comparison between FSL and P.452-16 at Betzdorf latitude

## Clutter Loss

Depending on the environment, it may make sense to consider additional attenuation due to building and or vegetation between the WBB BS and the FSS ES.  When looking at the configuration of different locations of FSS ES in Europe, these stations can be partially surrounded by bench of vegetation or building, but certainly not at 360° as there are also directions and areas which miss vegetation to attenuate the interfering signal. Therefore, while assuming no clutter at all may guide to worst case configuration, although possible, considering double clutter all the time reveal to be unrealistic assumptions. This is confirmed in the figure below which shows possible WBB LMP site around one FSS ES site of SES in Luxembourg. We can see cases where there is line of sight or clutter in the link between the WBBB LMP BS and the FSS ES.

In this study, we will see the impact of no clutter, one clutter and two clutters, so that it can be appreciated how the presence of clutter on a specific direction may impact positively the coexistence between WBB LMP and FSS ES. As such, when considered and as justified previously considering the 50% time assumption, the clutter will contribute to an additional 31 dB of attenuation to the interfering signal power in urban case.

Figure 6: Configuration of line of sight and clutter around an FSS ES in Luxembourg

## static MCL analysis

All impacting parameters identified in the previous sections are put together to meet the required additional attenuation for the WBB LMP BS to not exceed the FSS ES protection criterion. Depending on the assumption considered, the corresponding separation distance may vary. In order to find the minimum separation distance to avoid exceeding the FSS ES protection criterion, the following equation is computed :

Dmin=d |  IWBB_S=IMAX

The table below presents different assumptions under which the minimum separation distance required for the protection of FSS Earth station is calculated.

Table 2: Different assumptions to estimate the minimum separation distance

| Parameter | Case #1 | Case #2 | Case #3 | Case #4 | Case #5 |
| --- | --- | --- | --- | --- | --- |
| WBB LMP BS PTx (dBm/40MHz) | 21 | 35 | 35 | 37 | 37 |
| BS antenna gain (dBi) F1336_5 | 6 | 10 | 10 | 10 | 10 |
| Antenna height (m) | BS:10 ES:10 | BS:15 ES:10 | BS:15 ES:10 | BS:30 ES:10 | BS:30 ES:10 |
| BS Antenna discrimination1 (dB) | EL: 0 AZ : 0°,180° | EL: 6 AZ: 0°,180° | EL: 6 AZ: 0°,180° | EL: 6 AZ: 0°, 180° | EL: 6 AZ : 0°, 180° |
| Clutter Loss (dB) | 0 | 0 |  | 0 |  |
|  | 22* | 22*, 29 | 44*, 58 | 29 | 58 |
|  | 31 | 31 | 62 | 31 | 62 |
| FSL (Yes/No) | Yes |  |  |  |  |
| P45216 (Yes/No) | Yes | Yes | Yes | Yes | Yes |
| 1: Discrimination implies simultaneously azimuth, elevation, mechanical tilt, and electrical tilt. Attenuation from Max gain. EL in the table corresponds to the downtilt * Incremental |  |  |  |  |  |

We focus first on the long-term interference. It should be observed that Case #1 is Low Power BS and Case #2 and above are Medium Power BS. Looking at the summary table above, we can observe that two elements differentiate the two assumptions, with the MP BS having 14 dB more in transmit power and 4 dB more in antenna gain. This leads to the major difference between the two assumptions in the total EIRP with a difference of 18dB so that EIRPMP = EIRPLP + 18dB.

When the sensitive case of maximum power is considered with an additional 2 dB in the transmitted power, the difference between low and medium power systems is 20 dB, with EIRPMP = EIRPLP + 20dB.

## Low power base station

Principle of the single entry analysis

- For each configuration, results are expressed in terms of separation distance between WBB LMP base station and FSS earth station to meet the long-term protection criterion. Although the Free Space path loss (FSL) model is used for the calibration of the study, the analysis is performed using the P.45216 channel model which shows more reliable results for long distances from the victim FSS receiver. The associated results with this channel model are presented in the following figures.

- Case #1: Low Power BS

Under that configuration, the transmit power of the WBB LMP BS is PTx = 21 dBm/40MHz, what corresponds to 27 dBm/40MHz EIRP with the 6 dBi antenna gain. This is the low power BS maximum power under the UK/Norway framework.

The basic assumption to consider is p.45216 channel model with line-of-sight configuration, LMP BS antenna down-tilt and no clutter in the link. The result of such configuration is presented below for different FSS ES elevation angles and the WBB LMP BS pointing towards the FSS ES (Azimuth = 0°) or the ES located at the BS backside (Azimuth = 180°).

When adding a clutter in the link, the received interference and the required separation distance for different FSS elevation angles are reduced and the evolutions for the sub-urban and urban environments are illustrated below.

Ptx = 21 dBm, P.45216, no clutter, Azimuth = 0°(left) and 180°(right) between BS and ES

Elevation 5.0 degree is incremental

Figure 7: Received interference for LP BS with P.45216 and direct visibility between antennas

- Ptx = 21 dBm, 1 Clutter=29 dB (sub-urban), Diam = 4.5 m, Azimuth = 0° (left) and Azimuth = 180° (right)

- Ptx = 21 dBm, 1 Clutter=31 dB (urban), Diam = 4.5 m, Azimuth = 0° (left) and Azimuth = 180° (right)

- Elevation 5.0 degree is incremental

Figure 8: Received interference for Low Power BS with P.45216 and clutter in the link

- All the results illustrated previously are summarized in the table below

Table 3: Summary of the results for low power BS and the different assumptions with P.45216

| Configuration | Ptx = 21 dBm, P.45216 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FSS elevation |  | 5° Incremental | 10° | 20° | 30° | 40° | 50° |
| No clutter | Az = 0° | 32.6 km | 26.9 km | 21.3 km | 18.5 km | 16.6 km | 15.4 km |
|  | Az = 180° | 15.9 km | 11.5 km | 4.6 km | 2.75 km | 1.9 km | 1.51 km |
| Clutter = 29dB | Az = 0° | 12.8 km | 6 km | 2.5 km | 1.5 km | 1 km | 0.8 km |
|  | Az = 180° | 0.9 km | 0.37 km | 0.16 km |  |  |  |
| Clutter = 31dB | Az = 0° | 11.6 km | 4.8 km | 1.95 km | 1.2 km | 0.8 km | 0.6 km |
|  | Az = 180° | 0.71 km | 0.3 km | 0.13 km |  |  |  |

- Summary of the results

For each environment, results are expressed in terms of separation distance between WBB LMP base station and FSS earth station to meet the long-term protection criterion. These separation distances are assessed regarding the FSS earth station elevation angle and additionally azimuth between earth station and single base station.

The Free Space path loss (FSL) model is used for the calibration of the analysis, as we showed earlier that at a certain distance and depending on the assumptions, the deviation from the P452 model might lead to unrealistic results above a certain distance. However, for distances below 15km (exact number may depends on parameters such as antenna height and location of the station) the FSL is proposed to be used for fast assessment. It is also used to appreciating the single contribution of specific parameters to the potential interference as opposed to avoid the mutual and simultaneous aggregation of contributions from multiple parameters. The two cases of FSL and P.45216 lead to similar results and observations.  From the figures and tables above, it can be concluded that avoiding pointing the BS towards the FSS ES so that the FSS ES is located in the backside of the BS (180°) reduces considerably the required separation distance. At the reference locations of 30° elevation, if compared with the no clutter case the required separation distance is reduced to 2.75 km (85.1% reduction) when shifting from direct to backside pointing from the LMP BS.

It can also be extracted from the analysis that the presence of clutter in the link provides attenuation to the signal and allows having separation distance which can be managed, acknowledging that it cannot be assumed that there will always be 2 clutters in the link between LMP BS and FSS ES. At the reference locations of 30° elevation, when 1 clutter is considered and compared to the no clutter case, the required separation distance is reduced to 1.5 km (91.9% reduction) in sub-urban case and 1.2 km (93.5% reduction) in urban case.

## Medium Power Base station

The previous analysis with free space loss was useful to spotlight each parameter to see how it impacts the interference received by the FSS ES and define the theoretical separation distance required to not exceed the ES protection criteria. It was clarified that although the P.45216 channel model would be more realistic, the free space channel model gives an accurate assessment up to 15 km far from the FSS ES.

The following analyses are reconducted using P.45216 without terrain model to get the complete picture of the requirements in terms of separation distance between BS and ES for the different elevation angles of the ES. This assessment addresses cases #2 to #5 of the table of assumptions.

A sensitive case with low attenuation from clutter (22 dB) is assessed to see a middle stage impact from clutter, given that the 2dB difference between urban and sub-urban values may have in some cases a limited difference in the result.

- Case #2 & #3 : Medium Power BS and P.45216

Under these two scenarios, the transmit power of the WBB LMP BS is PTx = 35 dBm/40MHz, what corresponds to 45 dBm/40MHz EIRP with the 10 dBi antenna gain. This is equivalent to 49 dBm/100MHz which is the maximum authorized EIRP under the UK/Norway baseline framework. Case #2 considers no clutter, sub-urban clutter of 22 dB and 29 dB and urban clutter of 31 dB. Case #3 considers sub-urban double clutter of 44 dB and 58 dB and urban double clutter of 62 dB. It is recalled that those scenarios are 18 dB above LP BS EIRP.

No clutter in the link

| Ptx=35 dBm, Clutter = 0 dB, Diam = 4.5 m, P.45216 , Azimuth = 0° (left) and Azimuth = 180° (right) |  |
| --- | --- |
| Elevation 5.0 degree is incremental | Elevation 5.0 degree is incremental |

Figure 9: Required separation distance for P452 with Medium Power BS and no clutter.

At the reference 30° elevation ES, the separation distance required to protect the FSS is 30 km when the BS is pointing at the ES, and 17.5 km when the BS sees the ES from the backside. At 10° elevation for the ES, the required separation distances are respectively 39 km for the pointing case and 25.4 km fom the backside case.

It could be concluded that when the position of the FSS ES is know a coordination distance of 40 km could be defined, beyond which LMP BS can be deployed with no restriction and within which specific measures need to be implemented on a case by case basis.

One clutter in the link

The following figures present the impact of the presence of a clutter in the link between BS and ES for both urban and suburban cases.

| Ptx=35 dBm, Clutter = 22 dB, Diam = 4.5 m, P.45216 , Azimuth = 0° (left) and Azimuth = 180° (right) |  |
| --- | --- |
| Elevation 5.0 degree is incremental | Elevation 5.0 degree is incremental |

Figure 10: Required separation distance for P452 with Medium Power BS and 1 reduced clutter loss.

| Ptx=35 dBm, Clutter = 29 dB, Diam = 4.5 m, P.45216 , Azimuth = 0° (left) and Azimuth = 180° (right) |  |
| --- | --- |
| Elevation 5.0 degree is incremental | Elevation 5.0 degree is incremental |

Figure 11: Required separation distance for P452 with Medium Power BS and 1 suburban clutter.

| Ptx=35 dBm, Clutter = 31 dB, Diam = 4.5 m, P.45216 , Azimuth = 0° (left) and Azimuth = 180° (right) |  |
| --- | --- |
| Elevation 5.0 degree is incremental | Elevation 5.0 degree is incremental |

Figure 12: Required separation distance for P452 with Medium Power BS and 1 urban clutter.

Two clutters in the link (sensitivity analysis)

Ptx=35 dBm, 2 Clutters = 44 dB, Diam = 4.5 m, P.45216 , Azimuth = 0° (left) and Azimuth = 180° (right)

Elevation 5.0 degree is incremental

Figure 13: Separation distance for P452 with Medium Power BS and 2 reduced level clutters.

Ptx=35 dBm, 2 Clutters = 58 dB, Diam = 4.5 m, P.45216 , Azimuth = 0° (left) and Azimuth = 180° (right)

Elevation 5.0 degree is incremental

Figure 14: Required separation distance for P452 with Medium Power BS and 2 suburban clutters.

| Ptx=35 dBm, 2 Clutters = 62 dB, Diam = 4.5 m, P.45216 , Azimuth = 0° (left) and Azimuth = 180° (right) |
| --- |
| Elevation 5.0 degree is incremental |  |

Figure 15: Required separation distance for P452 with Medium Power BS and 2 urban clutters.

It is worth noting that :

The two clutters sensitivity analysis is provided purely for the purpose of the theoretical analysis to understand the potential improvement if such case were to happen at the local site. This configuration cannot be assumed to be representative of cases encountered in real deployment of system.

Due to the similarities of attenuation for FSL and P452 before the breaking point of P452, figures with clutters for P452 present results quasi-identical to those with FSL, with very small differences at the distances located in the small deviation area of the P452. This confirms the appropriateness of the free space model for fast assessment and coordination around the FSS ES.

These configurations assess the impact of clutter and we compare how the values assumed for the sub-urban and urban environments impact the required separation distance. To address further the presence of obstacles in the link, we evaluate the benefit of having a double clutter for the attenuation of the interference and how this may reduce further the required separation distance between BS and ES.

While this assessment is made, we couple it with the benefit of the BS not pointing towards the ES, recalling that the front to back ratio of the BS antenna is about 24 dB, which by itself is equivalent to a clutter. Therefore, full azimuthal discrimination (180°) coupled with 2 clutters is equivalent to having 3 clutters in the link, resulting then to extreme reduction of the interference level.

All the results obtained under these various configurations are summarized in the table below with the reference location of Luxembourg / The Netherlands in bold:

Table 4: Summary of the results for medium power BS and the different assumptions with P452

| Configuration | Ptx = 35 dBm, P.45216 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FSS elevation |  | 5° incremental | 10° | 20° | 30° | 40° | 50° |
| No clutter | Az = 0° | 44.6 km | 39.1 km | 33.6 km | 30.1 km | 27.25 km | 25.85 km |
|  | Az = 180° | 31.6 km | 25.4 km | 20.4 km | 17.5 km | 15.3 km | 13 km |
| Suburban 1 Clutter = 22dB | Az = 0° | 27.75 km | 22.5 km | 17.6 km | 14.3 km | 9.75 km | 7.7 km |
|  | Az = 180° | 15.7 km | 7.1 km | 2.9 km | 1.7 km | 1.2 km | 0.95 km |
| Suburban 1 Clutter = 29dB | Az = 0° | 22.9 km | 18 km | 10.4 km | 6.15 km | 4.2 km | 3.35 km |
|  | Az = 180° | 7.55 km | 3.05 km | 1.25 km | 0.75 km | 0.52 km | 0.42 km |
| Urban 1 Clutter = 31dB | Az = 0° | 21.5 km | 16.6 km | 8.2 km | 4.8 km | 3.3 km | 2.6 km |
|  | Az = 180° | 5.9 km | 2.4 km | 1 km | 0.6 km | 0.42 km | 0.33 km |
| 2 clutters sensitivity analysis |  |  |  |  |  |  |  |  |  |
| Suburban 2 Clutters=44dB | Az = 0° | 10.48 km | 4.22 km | 1.75 km | 1.02 km | 0.73 km | 0.56 km |
|  | Az = 180° | 1.27 km | 0.53 km | 0.22 km | 0.14 km |  |  |
| Suburban 2 Clutters=58dB | Az = 0° | 1.96 km | 0.81 km | 0.34 km | 0.2 km | 0.14 km | 0.11 km |
|  | Az = 180° | 0.25 km | 0.11 km |  |  |  |  |
| Urban 2 Clutters=62dB | Az = 0° | 1.23 km | 0.51 km | 0.21 km | 0.13 km |  |  |
|  | Az = 180° | 0.16 km |  |  |  |  |  |

The results show again that worst situation is obtained when the BS is pointing at the ES. So a clear guidance would be to avoid the MP BS be pointing at the FSS ES and having at least one visible obstacle (building, trees…) in the link between the BS and the ES.

- Case #4 & #5 : Incremental analysis Medium Power BS and P.45216

The previous analysis is reiterated considering this time the incremental approach with a MP BS transmitting at PTx = 37 dBm, which is 2 dB higher than the previous configurations. The results obtained are illustrated in the following figures.

Ptx=37 dBm, no Clutter, Diam = 4.5 m, P.45216 , Azimuth = 0° (left) and Azimuth = 180° (right)

Elevation 5.0 degree is incremental

Figure 16: Required separation distance for P452 with incremental BS and no clutter.

One clutter in the link

Ptx=37 dBm, 1 Clutter = 29 dB, Diam = 4.5 m, P.45216 , Azimuth = 0° (left) and Azimuth = 180° (right)

Ptx=37 dBm, 1 Clutter = 31 dB, Diam = 4.5 m, P.45216 , Azimuth = 0° (left) and Azimuth = 180° (right)

Elevation 5.0 degree is incremental

Figure 17: Required separation distance for P452 with incremental BS and 1 (sub)urban clutter.

Two clutters in the link (sensitivity analysis)

Ptx=37 dBm, 2 Clutters = 58 dB, Diam = 4.5 m, P.45216 , Azimuth = 0° (left) and Azimuth = 180° (right)

Ptx=37 dBm, 2 Clutters = 62 dB, Diam = 4.5 m, P.45216 , Azimuth = 0° (left) and Azimuth = 180° (right)

Elevation 5.0 degree is incremental

Figure 18: Required separation distance for P452 with incremental BS and 2 (sub)urban clutters.

All the results obtained under these various configurations are summarized in the table below with the reference location of Luxembourg/ The Netherlands at 30° elevation presented in bold:

Table 5: Summary of the results for incremental BS and the different assumptions with P452

| Configuration | Ptx = 37 dBm, P.45216 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FSS elevation |  | 5° incremental | 10° | 20° | 30° | 40° | 50° |
| No clutter | Az = 0° | 51.4 km | 45.7 km | 40 km | 36.6 km | 33.7 km | 31.9 km |
|  | Az = 180° | 38 km | 31.3 km | 25.8 km | 22.5 km | 20 km | 16.5 km |
| Suburban 1 Clutter = 29dB | Az = 0° | 30.3 km | 24.5 km | 13.3 km | 7.8 km | 5.4 km | 4.3 km |
|  | Az = 180° | 9.6 km | 3.9 km | 1.6 km | 0.95 km | 0.65 km | 0.55 km |
| Urban 1 Clutter = 31dB | Az = 0° | 27 km | 21.4 km | 10.4 km | 6.15 km | 4.2 km | 3.3 km |
|  | Az = 180° | 7.5 km | 3.05 km | 1.25 km | 0.75 km | 0.55 km | 0.42 km |
| 2 clutters sensitivity analysis |  |  |  |  |  |  |  |
| Suburban 2 Clutters = 58dB | Az = 0° | 2.49 km | 1.03 km | 0.43 km | 0.26 km | 0.18 km | 0.14 km |
|  | Az = 180° | 0.31 km | 0.13 km |  |  |  |  |
| Urban 2 Clutters = 62dB | Az = 0° | 1.55 km | 0.65 km | 0.27 km | 0.16 km | 0.12 km |  |
|  | Az = 180° | 0.2 km |  |  |  |  |  |

When adding 2 dB to the BS maximum transmit power retained from the reference UK/NOR framework to meet the value of the incremental approach of 37 dBm, we can observe that variation of the separation distance presents different values depending on the elevation angle. the separation distances to avoid harmful interference are increased for all cases.

For all the elevation angles above, the evolution of the required separation distances is presented in the following table. The statistical distribution exhibits a median increase of the separation distance of 27% and a mean increase of the separation distance of 26%.

Table 6: Evolution of the separation distance with the incremental approach

| Configuration | Evolution of separation distance from Ptx = 35 dBm Ptx = 37 dBm |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FSS elevation |  | 5° incremental | 10° | 20° | 30° | 40° | 50° |
| No clutter | Az = 0° | 15% | 17% | 19% | 22% | 24% | 23% |
|  | Az = 180° | 20% | 23% | 26% | 29% | 31% | 27% |
| Suburban 1 Clutter = 29dB | Az = 0° | 32% | 36% | 28% | 27% | 29% | 28% |
|  | Az = 180° | 27% | 28% | 28% | 27% | 25% | 31% |
| Urban 1 Clutter = 31dB | Az = 0° | 26% | 29% | 27% | 28% | 27% | 27% |
|  | Az = 180° | 27% | 27% | 25% | 25% | 31% | 27% |
| Suburban 2 Clutters = 58dB | Az = 0° | 27% | 27% | 26% | 30% | 29% | 27% |
|  | Az = 180° | 24% | 18% |  |  |  |  |
| Urban 2 Clutters = 62dB | Az = 0° | 26% | 27% | 29% | 23% |  |  |
|  | Az = 180° | 25% |  |  |  |  |  |

## Specific impact of different parameters

We assess the impact of different parameters on the results previously observed. The first element to account for is the applicability of the channel model.

- Difference between FSL and P.452

To evaluate the difference between FSL and P452 in the results, we consider no clutter, line of sight and BS pointing to ES (Azimuth = 0°) or having ES in backside (Azimuth = 180°).

| a) Ptx=35 dBm, Clutter = 0 dB, Azimuth = 0°, Diam = 4.5 m, FSL (left) and P.45216 (right) |  |
| --- | --- |
| b) Ptx=35 dBm, Clutter = 0 dB, Azimuth = 180°, Diam = 4.5 m, FSL (left) and P.45216 (right) |  |
| Elevation 5.0 degree is incremental | Elevation 5.0 degree is incremental |

Figure 19: Comparison between FSL and P452 with Medium Power BS and no clutter

These figures show how the P452 model shapes the level interference received, and we can clearly identify the breaking point around 15 km and from that point the way P452 reduces further the interference due to the rise of attenuation it provides compared to FSL.

- The Free Space Loss is then used for the assessment of the other parameters, to avoid mixing simultaneously the contribution of the parameter investigated with any impact of the channel model as it would be the case for the P.45216 from certain distances.

- Impact of FSS ES antenna diameter.

Different FSS ES antennas are used across Europe, and while some equipment has low antennas diameter of 1.2m, other have diameters higher than 4.5m and up to 16m or 32m. For the high antenna diameters, as provided in the FSS parameters list table, the system temperature is of 70K above 4.5m, leading to more sensible systems compared to antennas of 2.5m and less whose temperature is of 100K or more. For a 40MHz bandwidth system, the difference in system temperature leads to a difference of about 1.5 dB for the maximum interfering noise power Imax.

The consequence is that the required separation distance is decrease between WBB LMP BS and the FSS ES when reducing the ante size from 4.5m to 2.5m, as illustrated in the figures below. However, there is no change when the diameter is reduced from 2.5m to 1.2m as these two have same system temperature. Identically, there is no difference between the 4.5m and 8m antennas as they have same system temperature.

Given that a higher ES antenna diameter implies higher gain, this consolidates the fact that above 5° of elevation, there is no impact of the maximum ES antenna gain, as long as there is no change of the system temperature.

a) Ptx=35 dBm, Clutter = 29 dB, Azimuth = 0°, Diam = 1.2 m (left) and Diam= 2.5m (right)

b) Ptx=35 dBm, Clutter = 29 dB, Azimuth = 0°, Diam = 4.5 m (left) and Diam= 8m (right)

Elevation 5.0 degree is incremental

Figure 20: Impact of antenna diameter with Medium Power BS and 1 clutter

- Impact of LMP BS antenna down-tilt.

Antenna down-tilt of the LMP BS may have an impact depending on its value and the distance at which the FSS ES is far from the LMP BS. The impact in the received interference is evaluate for Case #2 urban (clutter = 29 dB and LMP Ptx =35 dBm). Three configurations of down-tilt are considered: 0°, 6°, 10°. When considering azimuth = 180° where the BS sees the ES from the back side, the down-tilt has no impact at all on the interference level received. This result is consistent given that from the backside of the LMP BS, the change of elevation from 0 to 10° has unsignificant effect on the gain towards the FSS ES.

Ptx=35 dBm, Clutter = 29 dB, Azimuth = 0°, Diam = 4.5 m, Down-tilt: 0° (left) and 10° (right)

Elevation 5.0 degree is incremental

Figure 21: Impact of antenna down-tilt with Medium Power BS, FSL, 1 clutter and Az= 0°

When considering azimuth = 0° where the BS points towards the ES, the required separation for 1 urban clutter of 29 dB with 6° down-tilt was of about 5.7 km at the reference point of 30° ES elevation (see results of Case #2 analysis).

Taking the 6° down-tilt as reference, reducing the down-tilt to 0° doubles the separation distance from 5.7 km to 11.5 km for an ES at elevation 30°, while the distance is reduced to 3.5 km for a 10° down-tilt of the BS.

Similarly, for a 30° elevation FSS ES, the required separation distance that is 153 km at 6° BS down-tilt is increased to more than 300 km for BS down-tilt of 0° and is reduced to 93° for a BS down-tilt of 10°.

For the impact of the LMP BS down-tilt, it can be concluded that it has a non-negligeable impact when the BS is pointing towards the FSS ES. A higher down-tilt reduces the impact of LMP BS transmissions into the victim FSS ES.

- Impact of LMP BS antenna height.

Antenna height of the LMP BS may have an impact depending on its value and the distance at which the FSS ES is far from the LMP BS. The impact in the received interference is evaluate for the case of sensitive (maximum) medium power BS with one urban clutter  (clutter = 31 dB and LMP Ptx =37 dBm). Three configurations of antenna height are considered: 10 m, 15 m, 25m and 35m. Results are presented in the figures below.

| a) Ptx=37 dBm, Clutter = 31 dB, Azimuth = 0°, Diam = 4.5 m, P.45216 , Alt = 10m (left) and 15m (right) |  |
| --- | --- |
| b) Ptx=37 dBm, Clutter = 31 dB, Azimuth = 0°, Diam = 4.5 m, P.45216 , Alt = 25m (left) and 35m (right) |  |
| Elevation 5.0 degree is incremental | Elevation 5.0 degree is incremental |

Figure 22: Impact of antenna height with Medium Power BS (max), P452, 1 clutter = 31dB and Az= 0°

The first observation that can be made is that results are exactly the same for elevation angles of 20° or more. It can be concluded that FSS ES of elevation angle higher than 20° are not sensitive to the BS antenna high up to the altitude simulated, i.e 35m.

For FSS ES of elevation below 20°, a higher LMP BS antenna altitude requires a longer separation distance, ranging from 15.5 km (10m altitude) to 24.3 km (35 m altitude) for an FSS ES of elevation of 10°. This is not neglectable for the constraint on protection or coordination around the FSS ES.

# Case study

Two sites have been considered for cases studies corresponding to existing location of FSS site in Europe. Site 1 is located at Betzdorf in Luxembourg and site 2 is located in Biddinghuizen in The Netherlands. Assuming a GSO satellite located at 19.2E in the previous section, these two sites were at the reference elevation angle around 30°. However, for a site like the one in Luxembourg which is the core site for a local operator (satellite infrastructure hub), allowing future deployment of FSS systems in the band means that nothing should preclude FSS earth stations to be pointing towards any GSO satellite on the total GSO arc. As such, any configuration of pointing of the FSS earth station should be considered when making the assessment of the potential interference received from any LMP BS located within its surroundings, so that the solution retained to allow the share use of the band by FSS and WBB is future proof.

In line with the above, a new configuration has been assumed to make a real case analysis based on the locations of existing sites, and their geographical environment including terrain model. The FSS earth stations at the reference sites are assumed to be pointing at any point towards the GSO arc. That corresponds to different off-axis angles towards a single BS, from each single earth station.

The limit of the pointing for each earth station is defined based on a common service level agreement in the industry which sets the minimum elevation angle to 5° so that a satellite location which leads to an elevation angle lower than 5° is ignored in this analysis. That results to the following graphics which illustrate the extreme satellites positions as seen by any FSS earth station at the reference sites, corresponding to longitudes of ±70° around the location of the sites.

Figure 23: Extreme visibility of GSO arc at ±70° longitude (5° elevation angle).

The overall target of these case studies is to assess how the local environment around the FSS ES affects the previous static analysis and at the end how the conducted investigation translates into field management to facilitate the shared use of the band by FSS and WBB LMP systems.

## Methodology

A rectangle area of [lat ± X°; lon ± Y°] is considered around the located FSS earth station. The FSS earth station is assumed to be pointing at any point of the Geo-Arc within lon±70° with a step of 1° in the variation of the satellite location. The minimum off-axis with respect to each position in the selected assessed area is calculated, and the corresponding FSS ES antenna gain towards each position of the BS is derived based on the considered antenna model. Reversely, the BS gain towards the FSS ES is also calculated, taking account of the antenna location, its height and downtilt (0° for low power and 6° for medium power). All LMP BS are assumed to be pointing in the direction of the FSS ES.

The path loss is then calculated between the FSS ES and each location of the BS, using P.45216 channel model and SRTM terrain model of 1 arc second (one data of terrain height above sea level every 30m). The received interference at the FSS ES from each BS is then calculated using the previously assessed parameters and any additional loss, as expressed in eq. 7. Finally, only all the BS positions which lead to a received interfering level above the FSS ES protection threshold are retained and presented.

When considering the short-term interference configuration, the model P.452-16 is used with the associated percentage of time of 0.005%, what leads to lower propagation loss values compared to the long-term case and consequently higher levels of the power of the interfering signal from WBB base station received at the FSS site. However, the interference trigger value associated to the short-term criterion is also increased compared to the long-term case and then the assessment consist in evaluating how much the risk of interference in increased under short-term interference configuration compared to the long-term interference configuration.

## Study case 1: LUXEMBURg site

The site is located at coordinates [lat, lon] = [49.6856°; 6.3245°]. Different power levels have been considered for the analysis. The terrain model in the selected area around the FSS ES is illustrated below.

Figure 24: Terrain height (in m) and path loss (in dB) from each BS to the FSS ES.

## Low power BS

Figure 25: Locations of low power BS exceeding the FSS ES I/N.

The green rectangle corresponds to the same exploration zone than the figure on the top, presented with a different scale.

## Medium power BS

Figure 26: Locations of medium power BS exceeding the FSS ES long-term I/N.

## Medium power incremental BS

The results for the incremental power are illustrated in the following figures for long-term and short-term protection criteria.

- Long-term criterion.

Figure 27: Locations of medium power incremental BS exceeding the FSS ES I/N.

## Comparison between short-term and long-term criteria

The analysis of the short-term interference for the low power BS and medium power incremental BS gives:

|  | a) LP BS locations exceeding the short-term I/N protection criteria |
| --- | --- |
|  | b) MP incremental BS locations exceeding the short-term I/N protection criteria |

Figure 28: Locations of LP and MP incremental BS exceeding the FSS ES short-term I/N.

For this case and compared to the long-term interference case, there is only a very limited difference with respect to the locations presenting a risk of interference from WBB MP BS into the FSS ES receiver. Environment between the stations plays a determining role in this result.

## Study case 2: the netherlands site

The site is located at coordinates [lat, lon] = [52.4629°; 5.7095°]. The terrain model in the selected area around the FSS ES is illustrated below.

Figure 29: Terrain height (in m)  and path loss (in dB) from each BS to the FSS ES.

The two extreme power levels have been considered for the analysis.

## Low power BS

The results for the incremental power are illustrated in the following figures for long-term and short-term protection criteria.

- Long-term criterion.

Figure 30: Locations of low power BS exceeding the FSS ES long-term I/N.

The green rectangle on corresponds to the same exploration zone than the illustration on the top.

- Short-term criterion.

Figure 31: Locations of low power BS long-term interference exceeding the FSS ES short-term I/N.

The analysis indicates that 54% of the spots exceeding the long-term protection limit are also exceeding the signal power limit associated to the short-term protection criterion. When making an analysis based on the short-term parameters in the propagation model, the result becomes as follows.

Figure 32: Locations of low power BS short-term interference exceeding the FSS ES short-term I/N.

## Medium power incremental BS

The results for the incremental power are illustrated in the following figure figures for long-term and short-term protection criteria.

- Long-term criterion.

Figure 33: Locations of medium power incremental BS exceeding the FSS ES long-term I/N.

The green rectangle corresponds to the explored area around the FSS site.

Short-term criterion

Figure 34: Locations of MP incremental BS long-term power exceeding the FSS ES short-term I/N.

The analysis indicates that 59.5% of the spots exceeding the long-term protection limit are also exceeding the signal power limit associated to the short-term protection criterion. When making an analysis based on the short-term parameters, the result becomes as follows.

Figure 35: Locations of MP incremental BS short-term power exceeding the FSS ES short-term I/N.

The previous Figure 33 reflects the results when applying the short-term criterion parameters to the analysis, through the percentage of time of 0.005% into the channel propagation model ITU-R P.452-16 and the associated received signal power level not to be exceeded from the transmitting WBB BS. The green rectangle delimits the explored area around the FSS site, therefore the illustration demonstrates that the possible locations exceeding the short-term protection criterion extend much beyond the limits of our analysis. For this case, as the terrain around the FSS site is almost flat, it has a limited impact in the signal attenuation and therefore there is a huge difference in the number of locations that are likely to create interference, compared to the case with short-term interference.

## Analysis of the cases studies

When analysing and comparing the results obtained for the two sites studies, the following elements can be derived:

- The results are consistent with the previous static analysis. This is more obvious when looking at the Netherlands site which stands in a pretty flat area compared to the Luxemburg site.

- When the BS is pointing in the direction of the FSS ES, the required separation is high if there is no obstacle to break the strength of the signal.

- The environment plays a major role in the potential interference received by the FSS ES. A fluctuating terrain like Luxembourg leads to more places which benefit from geographic configuration and possible hills that reduce the interference. At the opposite, a flat area such as the Netherlands has less obstacles and the receiving site faces more troubles with signals from the interfering BS.

- A coordination distance is needed which triggers site specific. It has to be adequately define to make it possible to deploy WBB LMP while minimizing the threat into FSS systems.

- Depending on the site configuration and environment between the two antennas, the consideration of the short-term interference could, in some cases, bring additional information when defining the right measures required to ensure the appropriate protection of the FSS site.

# Conclusion

The study presents the analysis of the potential interference from WBB LMP BS into victim FSS ES. It has been conducted in two phases:

- The static analysis that provides a clear picture of the various parameters impacting the interference received by the FSS ES from WBB LMP BS and allows identifying possible ways to mitigate the interference by applying site specific adjustments

- The statistical case study analysis which explores site-specific configuration and allows assessing the impact of the terrain and environment around the FSS ES in the received interference.

From the simulation performed it can be concluded that specific actions or measures need to be implemented to minimize the potential interference from WBB LMP stations into FSS ES. To facilitate the deployment of 5G system while limiting the impact into existing and future use of FSS systems, the following approaches may be considered, as appropriate:

- It is not recommended to position a WBB LMP BS antenna at any place where there is line of sight and direct visibility with an FSS earth station.  Blocks of vegetation or building should stand in the direct line between the two antennas.

- Avoiding pointing the WBB LMP towards the FSS earth station reduces the level of interference received at the FSS receiver, thus reducing the required separation distance. It is therefore recommended to position the WBB LMP BS antenna so that it sees the FSS earth station from the backside lobe or WBB LMP BS antenna not to point in the direction of FSS.

- FSS earth stations with higher antenna diameter have lower system temperature which reduces the maximum allowable interference by about 1.6dB. This results in higher protection distances compared to the 2.5m or lower antennas. The level of the increase of the protection distance with increasing size of antenna varies depending on the assumptions.

- Free space model can be used for fast assessment, when the distance between FSS ES and LMP BS is lower than the distance to the horizon for the assumed antenna heights (in this study this distance will correspond to15 km).

Based on the analysis (see Table 4), the results of this study suggest a coordination distance around an FSS ES location of 40km is suitable to protect FSS ES receivers. This corresponds to results for Medium Power BS limited to Ptx =35dBm/40 MHz (which corresponds EIRP=49 dBm/100 MHz with a 10 dBi antenna gain), model P.452-16, no clutter, flat terrain, FSS ES antenna diameter of 4.5m, azimuth discrimination of 0°, and FSS ES elevation angle of 10°. According to the location of LMP BS, further mitigation may be implemented, including among other possibilities ensuring non-pointing of the BS in the direction of the FSS ES, indoor BS, lower BS antenna height to include a building in the link, increasing the BS down-tilt, positioning a radom, reducing the BS transmit power…

Extreme cases have been assessed (Az= 0° and AZ=180°). As such, any dynamic analysis would provide results in-between those of these two cases. However, all the proposals made are applicable only when the location of the FSS ES is known. This study as well as the mitigation approaches presented before can be used to develop the toolbox expected from ECC PT1.

Some parameters in this table are similar to those in Report ITU-R S.2368-0